В унциях 1 Ounces  = 28.35 Grams
Acura
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ouces
	Type
	Ounces

	
	CL
	

	2001-2003
	All Engines
	R134a
	23.00
	PAG 46
	5.50

	1997-1999
	All Engines
	R134a
	26.00
	PAG 46
	5.50

	
	INTEGRA
	

	1994-2001
	All Engines
	R134a
	24.00
	PAG 46
	5.00

	1989-1993
	All Engines
	R12
	34.00
	Mineral
	5.00

	1987-1988
	All Engines
	R12
	33.00
	Mineral
	5.00

	
	LEGEND
	

	1994-1995
	All Engines
	R134a
	24.00
	PAG 46
	5.00

	1993
	All Engines
	R134a
	24.00
	PAG 46
	6.50

	1991-1992
	All Engines
	R12
	27.00
	Mineral
	6.50

	1989-1990
	All Engines
	R12
	30.00
	Mineral
	6.50

	1987-1988
	All Engines
	R12
	33.00
	Mineral
	6.50

	
	MDX
	

	2002-2004
	All Engines
	R134a
	25.50
	PAG 46
	5.00

	2001
	All Engines
	R134a
	27.00
	PAG 46
	5.50

	
	NSX
	

	1997-2004
	All Engines
	R134a
	30.00
	PAG 46
	5.50

	1993-1996
	All Engines
	R134a
	30.00
	PAG 46
	5.00

	1991-1992
	All Engines
	R12
	34.00
	Mineral
	5.00

	
	RL
	

	2002-2003
	All Engines
	R134a
	25.00
	PAG 46
	5.00

	1996-2001
	All Engines
	R134a
	26.00
	PAG 46
	5.00

	
	RSX
	

	2002-2004
	All Engines
	R134a
	18.00
	PAG 46
	5.50

	
	SLX
	

	1998-1999
	All Engines
	R134a
	21.00
	PAG 46
	5.00

	1996-1997
	All Engines
	R134a
	28.00
	PAG 46
	5.00

	
	TL
	

	1999-2003
	All Engines
	R134a
	23.00
	PAG 46
	5.50

	1995-1998
	All Engines
	R134a
	27.00
	PAG 46
	5.50

	
	VIGOR
	

	1992-1994
	All Engines
	R12
	30.00
	Mineral
	7.00


Audi
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	80 SERIES
	

	1989-1992
	80 ...
	R12
	37.00
	Mineral
	10.00

	
	90 SERIES
	

	1993-1995
	90 ...
	R134a
	23.00
	PAG 46
	8.50

	1989-1991
	90 ...
	R12
	37.00
	Mineral
	10.00

	
	100 SERIES
	

	1993-1994
	100 ...
	R134a
	26.50
	PAG 46
	8.50

	1989-1992
	100 ...
	R12
	38.00
	Mineral
	10.00

	
	200 SERIES
	

	1991
	200 ...
	R12
	38.00
	Mineral
	10.00

	
	A4 SERIES
	

	2000-2001
	A4 ...
	R134a
	24.00
	PAG 46
	9.00

	1996-1999
	A4 ...
	R134a
	23.00
	PAG 46
	8.50

	
	A6 SERIES
	

	2001
	A6 ...
	R134a
	24.00
	PAG 46
	8.50

	1998-2000
	A6 ...
	R134a
	28.00
	PAG 46
	8.50

	1996-1997
	A6 ...
	R134a
	21.00
	PAG 46
	8.50

	1995
	A6 ...
	R134a
	26.50
	PAG 46
	8.50

	
	A8 SERIES
	

	1999
	A8 ...
	R134a
	21.00
	PAG 46
	8.50

	1997-1998
	A8 ...
	R134a
	21.00
	PAG 46
	8.50

	
	CABRIOLET
	

	1995-1998
	All Engines
	R134a
	23.00
	PAG 46
	8.50

	
	COUPE QUATTRO
	

	1991
	All Engines
	R12
	37.00
	Mineral
	10.00

	
	S SERIES
	

	1995
	S6 ...
	R134a
	26.50
	PAG 46
	8.50

	1993-1994
	S4 ...
	R134a
	26.50
	PAG 46
	8.50

	1992
	S4 ...
	R12
	38.00
	Mineral
	10.00

	
	TT SERIES
	

	2000-2001
	TT ...
	R134a
	28.00
	PAG 46
	5.00

	
	V8 QUATTRO
	

	1993-1994
	All Engines
	R134a
	30.00
	PAG 46
	8.50

	1990-1992
	All Engines
	R12
	39.00
	Mineral
	10.00


BMW
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	318 SERIES
	

	1993-1999
	All Engines - Flat Tube Condenser ...
	R134a
	29.00
	PAG 100
	6.00

	
	All Engines - Round Tube Condenser ...
	R134a
	36.00
	PAG 100
	6.00

	1991-1992
	All Engines - w/ Behr Compressor ...
	R12
	42.50
	Mineral
	6.75

	
	All Engines - w/ Bosch Compressor ...
	R12
	42.50
	Mineral
	10.00

	
	323 SERIES
	

	2000
	All Engines - From 9/98 ...
	R134a
	29.50
	PAG 100
	5.50

	1993-1999
	All Engines - Flat Tube Condenser ...
	R134a
	29.00
	PAG 100
	6.00

	
	All Engines - Round Tube Condenser ...
	R134a
	36.00
	PAG 100
	6.00

	1991-1992
	All Engines - w/ Behr Compressor ...
	R12
	42.50
	Mineral
	6.75

	
	All Engines - w/ Bosch Compressor ...
	R12
	42.50
	Mineral
	10.00

	
	325 SERIES
	

	2001
	All Engines
	R134a
	25.50
	PAG 100
	5.50

	1993-1996
	All Engines
	R134a
	36.00
	PAG 100
	6.00

	1991-1992
	All Engines - w/ Behr Compressor ...
	R12
	42.50
	Mineral
	6.75

	
	All Engines - w/ Bosch Compressor ...
	R12
	42.00
	Mineral
	10.00

	1989-1990
	All Engines - w/ Behr Compressor ...
	R12
	34.50
	Mineral
	6.75

	
	All Engines - w/ Bosch Compressor ...
	R12
	34.50
	Mineral
	10.00

	1986-1987
	All Engines
	R12
	33.50
	Mineral
	5.75

	
	328 SERIES
	

	1996-2000
	All Engines
	R134a
	36.00
	PAG 100
	6.00

	
	525 SERIES
	

	1993-1995
	525I - Flat Tube Condenser ...
	R134a
	51.00
	PAG 100
	6.00

	
	525IT - Flat Tube Condenser ...
	R134a
	51.00
	PAG 100
	6.00

	
	525I - Round Tube Condenser ...
	R134a
	53.00
	PAG 100
	6.00

	
	525IT - Round Tube Condenser ...
	R134a
	53.00
	PAG 100
	6.00

	1989-1992
	525I - w/ Behr Compressor ...
	R12
	68.00
	Mineral
	6.75

	
	525I - w/ Bosch Compressor ...
	R12
	68.00
	Mineral
	10.00

	
	528 SERIES
	

	2000
	528I ...
	R134a
	43.00
	PAG 100
	6.00

	1999
	528I ...
	R134a
	44.00
	PAG 100
	6.00

	1997-1998
	528I ...
	R134a
	43.00
	PAG 100
	6.00

	
	530 SERIES
	

	1994-1995
	530I - Flat Tube Condenser ...
	R134a
	51.00
	PAG 100
	6.00

	
	530IT - Flat Tube Condenser ...
	R134a
	51.00
	PAG 100
	6.00

	
	530I - Round Tube Condenser ...
	R134a
	53.00
	PAG 100
	6.00

	
	530IT - Round Tube Condenser ...
	R134a
	53.00
	PAG 100
	6.00

	
	535 SERIES
	

	1993-1995
	535I - Flat Tube Condenser ...
	R134a
	51.00
	PAG 100
	6.00

	
	535I - Round Tube Condenser ...
	R134a
	53.00
	PAG 100
	6.00

	1989-1992
	535I - w/ Behr Compressor ...
	R12
	68.00
	Mineral
	6.75

	1989-1992
	535I - w/ Bosch Compressor ...
	R12
	68.00
	Mineral
	10.00

	
	540 SERIES
	

	1999-2000
	540I ...
	R134a
	44.00
	PAG 100
	6.00

	1997-1998
	540I ...
	R134a
	43.00
	PAG 100
	6.00

	1994-1995
	540I - Flat Tube Condenser ...
	R134a
	51.00
	PAG 100
	6.00

	
	540I - Round Tube Condenser ...
	R134a
	53.00
	PAG 100
	6.00

	
	635 SERIES
	

	1989
	All Engines w/ Front & Rear AC ...
	R12
	63.50
	Mineral
	5.75

	
	All Engines w/ Front AC Only ...
	R12
	39.00
	Mineral
	5.75

	
	735 SERIES
	

	1992
	All Engines w/ Front & Rear AC 4/92-12/92 ...
	R134a
	60.00
	PAG 100
	6.00

	
	All Engines w/ Front & Rear AC To 3/92 - w/ Behr Compressor ...
	R12
	73.00
	Mineral
	6.75

	
	All Engines w/ Front AC Only To 3/92 - w/ Behr Compressor ...
	R12
	68.00
	Mineral
	6.75

	
	All Engines w/ Front & Rear AC To 3/92 - w/ Bosch Compressor ...
	R12
	73.00
	Mineral
	10.00

	
	All Engines w/ Front AC Only To 3/92 - w/ Bosch Compressor ...
	R12
	68.00
	Mineral
	10.00

	
	All Engines 4/92-12/92 - w/ Denso Compressor ...
	R134a
	55.00
	PAG 100
	5.50

	
	All Engines 4/92-12/92 - w/ Seiko Seiki Compressor ...
	R134a
	55.00
	PAG 100
	6.00

	1990-1991
	All Engines w/ Front & Rear AC To 3/92 - w/ Behr Compressor ...
	R12
	73.00
	Mineral
	6.75

	
	All Engines w/ Front AC Only To 3/92 - w/ Behr Compressor ...
	R12
	68.00
	Mineral
	6.75

	
	All Engines w/ Front & Rear AC To 3/92 - w/ Bosch Compressor ...
	R12
	73.00
	Mineral
	10.00

	
	All Engines w/ Front AC Only To 3/92 - w/ Bosch Compressor ...
	R12
	68.00
	Mineral
	10.00

	1989
	All Engines w/ Front & Rear AC To 3/92 - w/ Behr Compressor ...
	R12
	73.00
	Mineral
	6.75

	
	All Engines To 3/92 - w/ Behr Compressor ...
	R12
	68.00
	Mineral
	6.75

	
	All Engines w/ Front & Rear AC To 3/92 - w/ Bosch Compressor ...
	R12
	73.00
	Mineral
	10.00

	
	All Engines To 3/92 - w/ Bosch Compressor ...
	R12
	68.00
	Mineral
	10.00

	
	740 SERIES
	

	1999-2000
	All Engines
	R134a
	55.00
	PAG 100
	6.00

	1995-1998
	All Engines - w/ Denso Compressor ...
	R134a
	42.50
	PAG 100
	5.50

	
	All Engines - w/ Seiko Seiki Compressor ...
	R134a
	42.50
	PAG 100
	6.00

	1993-1994
	All Engines w/ Front & Rear AC ...
	R134a
	60.00
	PAG 100
	6.00

	
	All Engines w/ Front AC Only ...
	R134a
	55.00
	PAG 100
	6.00

	
	750 SERIES
	

	1999-2000
	All Engines
	R134a
	43.00
	PAG 100
	6.50

	1998
	All Engines w/ Front & Rear AC ...
	R134a
	42.50
	PAG 100
	6.00

	
	All Engines w/ Front AC Only - w/ Denso Compressor ...
	R134a
	42.50
	PAG 100
	5.50

	
	All Engines w/ Front AC Only - w/ Seiko Seiki Compressor ...
	R134a
	42.50
	PAG 100
	6.00

	1997
	All Engines w/ Rear AC ...
	R134a
	42.50
	PAG 100
	6.00

	
	All Engines w/ Front & Rear AC - w/ Denso Compressor ...
	R134a
	42.50
	PAG 100
	5.50

	
	All Engines w/ Front AC Only - w/ Seiko Seiki Compressor ...
	R134a
	42.50
	PAG 100
	6.00

	1995-1996
	All Engines w/ Front & Rear AC ...
	R134a
	42.50
	PAG 100
	6.00

	
	All Engines w/ Front AC Only - w/ Denso Compressor ...
	R134a
	42.50
	PAG 100
	5.50

	
	All Engines w/ Front AC Only - w/ Seiko Seiki Compressor ...
	R134a
	42.50
	PAG 100
	6.00

	1993-1994
	All Engines w/ Front & Rear AC ...
	R134a
	60.00
	PAG 100
	6.00

	
	All Engines w/ Front AC Only - w/ Denso Compressor ...
	R134a
	55.00
	PAG 100
	5.50

	
	All Engines w/ Front AC Only - w/ Seiko Seiki Compressor ...
	R134a
	55.00
	PAG 100
	6.00

	1989-1991
	All Engines w/ Front & Rear AC - w/ Behr Compressor ...
	R12
	73.00
	Mineral
	6.75

	
	All Engines w/ Front AC Only - w/ Behr Compressor ...
	R12
	68.00
	Mineral
	6.75

	
	All Engines w/ Front & Rear AC - w/ Bosch Compressor ...
	R12
	73.00
	Mineral
	10.00

	
	All Engines w/ Front AC Only - w/ Bosch Compressor ...
	R12
	68.00
	Mineral
	10.00

	
	840 SERIES
	

	1994-1998
	All Engines - w/ Denso Compressor ...
	R134a
	55.00
	PAG 100
	5.50

	
	All Engines - w/ Seiko Seiki Compressor ...
	R134a
	55.00
	PAG 100
	6.00

	
	850 SERIES
	

	1993-1997
	All Engines - w/ Denso Compressor ...
	R134a
	55.00
	PAG 100
	5.50

	
	All Engines - w/ Seiko Seiki Compressor ...
	R134a
	55.00
	PAG 100
	6.00

	1990-1992
	All Engines - w/ Behr Compressor ...
	R12
	68.00
	Mineral
	6.75

	
	All Engines - w/ Bosch Compressor ...
	R12
	68.00
	Mineral
	10.00

	
	M SERIES
	

	1995-1999
	All Engines - Flat Tube Condenser ...
	R134a
	29.00
	PAG 100
	5.00

	
	All Engines - Round Tube Condenser ...
	R134a
	36.00
	PAG 100
	5.00

	1993
	All Engines - Flat Tube Condenser ...
	R134a
	51.00
	PAG 100
	6.00

	
	All Engines - Round Tube Condenser ...
	R134a
	53.00
	PAG 100
	6.00

	1992
	All Engines
	R12
	53.00
	Mineral
	6.75

	1991
	4 Cyl. - w/ Behr Compressor ...
	R12
	31.00
	Mineral
	6.75

	
	4 Cyl. - w/ Bosch Compressor ...
	R12
	31.00
	Mineral
	10.00

	
	6 Cyl. ..
	R12
	53.00
	Mineral
	6.75

	1989-1990
	All Engines - w/ Behr Compressor ...
	R12
	31.00
	Mineral
	6.75

	
	All Engines - w/ Bosch Compressor ...
	R12
	31.00
	Mineral
	10.00

	
	X SERIES
	

	2001
	X5 ...
	R134a
	16.00
	PAG 100
	--

	
	Z SERIES
	

	1999-2000
	Z3 - Flat Tube Condenser ...
	R134a
	29.00
	PAG 100
	6.00

	
	Z3 - Round Tube Condenser ...
	R134a
	36.00
	PAG 100
	6.00


Buick
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	CENTURY SEDAN / COUPE
	

	2003-2004
	All Engines
	R134a
	36.00
	PAG 150
	9.00

	1997-2002
	All Engines
	R134a
	36.00
	PAG 150
	8.00

	1994-1996
	All Engines
	R134a
	28.00
	PAG 150
	8.00

	1993
	All Engines
	R12
	38.00
	Mineral
	8.00

	1992
	All Engines
	R12
	40.00
	Mineral
	8.00

	1985-1991
	All Engines
	R12
	44.00
	Mineral
	8.00

	
	ELECTRA SEDAN / COUPE
	

	1989-1990
	All Engines
	R12
	38.50
	Mineral
	8.00

	1988
	All Engines
	R12
	36.00
	Mineral
	8.00

	1986-1987
	All Engines
	R12
	46.00
	Mineral
	8.00

	1985
	All Engines
	R12
	56.00
	Mineral
	8.00

	
	LESABRE SEDAN / COUPE
	

	2003-2004
	All Engines
	R134a
	35.00
	PAG 150
	9.00

	1998-2002
	All Engines
	R134a
	32.00
	PAG 150
	9.00

	1996-1997
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1994-1995
	All Engines
	R134a
	39.00
	PAG 150
	8.00

	1989-1993
	All Engines
	R12
	38.50
	Mineral
	8.00

	1988
	All Engines
	R12
	53.00
	Mineral
	8.00

	1986-1987
	All Engines
	R12
	38.00
	Mineral
	8.00

	1985
	All Engines
	R12
	56.00
	Mineral
	8.00

	
	PARK AVENUE
	

	2000-2004
	All Engines
	R134a
	35.00
	PAG 150
	9.00

	1997-1999
	All Engines
	R134a
	32.00
	PAG 150
	9.00

	1996
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1994-1995
	All Engines
	R134a
	39.00
	PAG 150
	8.00

	1992-1993
	All Engines
	R12
	38.50
	Mineral
	8.00

	1991
	All Engines
	R12
	39.00
	Mineral
	8.00

	
	RAINIER
	

	2004
	All Engines - w/ Rear AC ...
	R134a
	48.00
	PAG 150
	8.50

	
	All Engines - w/o Rear AC ...
	R134a
	29.00
	PAG 150
	7.50

	
	REATTA
	

	1989-1991
	All Engines
	R12
	38.50
	Mineral
	8.00

	
	REGAL SEDAN / COUPE
	

	1997-2004
	All Engines
	R134a
	36.00
	PAG 150
	8.00

	1994-1996
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1988-1993
	All Engines
	R12
	36.00
	Mineral
	8.00

	1985-1987
	All Engines
	R12
	52.00
	Mineral
	8.00

	
	RENDEZVOUS
	

	2001-2004
	All Engines
	R134a
	27.00
	PAG 46
	7.00

	
	RIVIERA
	

	1997-1999
	All Engines
	R134a
	32.00
	PAG 150
	9.00

	1995-1996
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1989-1993
	All Engines
	R12
	38.00
	Mineral
	8.00

	1985
	All Engines
	R12
	52.00
	Mineral
	7.00

	
	ROADMASTER
	

	1994-1996
	All Engines
	R134a
	28.00
	PAG 150
	8.00

	1991-1993
	All Engines
	R12
	50.00
	Mineral
	6.00

	
	SKYHAWK
	

	1989
	All Engines
	R12
	36.00
	Mineral
	8.00

	
	SKYLARK
	

	1995-1998
	All Engines
	R134a
	36.00
	PAG 150
	9.00

	1994
	All Engines
	R134a
	36.00
	PAG 150
	8.00

	1993
	All Engines
	R12
	42.00
	Mineral
	8.00

	1992
	All Engines
	R12
	42.50
	Mineral
	8.00

	1989-1991
	All Engines
	R12
	36.00
	Mineral
	8.00


Cadillac
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	ALLANTE
	

	1987-1993
	All Engines
	R12
	36.00
	Mineral
	8.00

	
	BROUGHAM
	

	1991-1992
	All Engines
	R12
	49.50
	Mineral
	8.00

	1987-1990
	All Engines
	R12
	56.00
	Mineral
	8.00

	1985-1986
	All Engines
	R12
	56.00
	Mineral
	7.00

	
	CATERA
	

	1999-2001
	All Engines
	R134a
	41.00
	PAG 150
	8.00

	1997-1998
	All Engines
	R134a
	41.50
	PAG 150
	9.00

	
	CIMARRON
	

	1986-1988
	All Engines
	R12
	36.00
	Mineral
	8.00

	
	CONCOURS
	

	1996-1999
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1994-1995
	All Engines
	R134a
	39.00
	PAG 150
	8.00

	
	CTS
	

	2003-2004
	All Engines
	R134a
	21.00
	PAG 46
	4.75

	
	DEVILLE
	

	2002-2004
	All Engines
	R134a
	35.00
	PAG 46
	9.00

	2000-2001
	All Engines
	R134a
	32.00
	PAG 46
	9.00

	1998-1999
	All Engines
	R134a
	32.00
	PAG 150
	9.00

	1994-1997
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1991-1993
	All Engines
	R12
	39.00
	Mineral
	8.00

	1989-1990
	All Engines
	R12
	46.00
	Mineral
	8.00

	1988
	All Engines
	R12
	38.00
	Mineral
	8.00

	1985-1987
	All Engines
	R12
	46.00
	Mineral
	8.00

	
	ELDORADO
	

	1998-2002
	All Engines
	R134a
	32.00
	PAG 150
	9.00

	1994-1997
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1991-1993
	All Engines
	R12
	38.00
	Mineral
	8.00

	1989-1990
	All Engines
	R12
	38.50
	Mineral
	8.00

	1988
	All Engines
	R12
	44.00
	Mineral
	8.00

	1985
	All Engines
	R12
	56.00
	Mineral
	7.00

	
	ESCALADE
	

	2003-2004
	All Engines w/ Front & Rear AC ...
	R134a
	43.00
	PAG 46
	11.00

	
	All Engines w/ Front AC Only ...
	R134a
	25.50
	PAG 46
	8.00

	
	ExT w/ Front AC Only ...
	R134a
	25.50
	PAG 46
	8.00

	
	ExT w/ Front & Rear AC ...
	R134a
	43.00
	PAG 46
	11.00

	
	Exv w/ Front & Rear AC ...
	R134a
	48.00
	PAG 46
	11.00

	2003-2004
	Exv w/ Front AC Only ...
	R134a
	25.50
	PAG 46
	8.00

	2002
	All Engines w/ Front & Rear AC ...
	R134a
	42.00
	PAG 46
	11.00

	
	All Engines w/ Front AC Only ...
	R134a
	28.00
	PAG 46
	8.00

	
	ExT w/ Front AC Only ...
	R134a
	28.00
	PAG 46
	8.00

	
	ExT w/ Front & Rear AC ...
	R134a
	42.00
	PAG 46
	11.00

	2000
	All Engines w/ Front AC Only - w/ Denso Compressor ...
	R134a
	32.00
	PAG 46
	8.00

	
	All Engines w/ Front & Rear AC - w/ Denso Compressor ...
	R134a
	48.00
	PAG 46
	11.00

	1999
	All Engines w/ Front & Rear AC - w/ HT6 Compressor ...
	R134a
	64.00
	PAG 150
	11.00

	
	All Engines w/ Front AC Only - w/ HT6 Compressor ...
	R134a
	36.00
	PAG 150
	8.00

	
	FLEETWOOD
	

	1994-1996
	All Engines RWD ...
	R134a
	28.00
	PAG 150
	8.00

	1993
	All Engines RWD ...
	R12
	50.00
	Mineral
	8.00

	1991-1992
	All Engines FWD ...
	R12
	39.00
	Mineral
	8.00

	1989-1990
	All Engines FWD ...
	R12
	46.00
	Mineral
	8.00

	
	SEVILLE
	

	2001-2003
	All Engines - Mitsubishi Compressor ...
	R134a
	35.00
	PAG 46
	9.00

	1999-2000
	All Engines - Mitsubishi Compressor ...
	R134a
	32.00
	PAG 46
	9.00

	1998
	All Engines - HD6 Compressor ...
	R134a
	32.00
	PAG 150
	9.00

	
	All Engines - Mitsubishi Compressor ...
	R134a
	32.00
	PAG 46
	9.00

	1994-1997
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1991-1993
	All Engines
	R12
	38.00
	Mineral
	8.00

	1989-1990
	All Engines
	R12
	38.50
	Mineral
	8.00

	
	SRX
	

	2004
	All Engines w/ Front & Rear AC ...
	R134a
	28.00
	PAG 46
	7.00

	
	All Engines w/ Front AC Only ...
	R134a
	21.00
	PAG 46
	7.00

	
	XLR
	

	2004
	All Engines
	R134a
	22.00
	PAG 46
	5.00


Chevrolet
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	AVEO
	

	2004
	All Engines
	R134a
	35.00
	PAG 46
	8.75

	
	BERETTA
	

	1994-1996
	All Engines
	R134a
	36.00
	PAG 150
	9.00

	1992-1993
	All Engines
	R12
	42.00
	Mineral
	8.00

	1991
	All Engines
	R12
	36.00
	Mineral
	8.00

	1988-1990
	All Engines
	R12
	44.00
	Mineral
	8.00

	
	CAMARO
	

	1998-2002
	All Engines
	R134a
	24.00
	PAG 150
	9.00

	1997
	6 Cyl. 3.8 Eng. ..
	R134a
	26.00
	PAG 150
	8.00

	
	8 Cyl. 5.7 Eng. ..
	R134a
	32.00
	PAG 150
	8.00

	1996
	6 Cyl. 3.8 Eng. ..
	R134a
	28.00
	PAG 150
	8.00

	
	8 Cyl. 5.7 Eng. ..
	R134a
	32.00
	PAG 150
	8.00

	1993-1995
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1989-1992
	All Engines
	R12
	36.00
	Mineral
	8.00

	1985-1987
	All Engines
	R12
	48.00
	Mineral
	8.00

	
	CAPRICE
	

	1994-1996
	All Engines
	R134a
	28.00
	PAG 150
	8.00

	1991-1993
	All Engines
	R12
	50.00
	Mineral
	8.00

	1989-1990
	All Engines
	R12
	44.00
	Mineral
	8.00

	1985-1988
	All Engines
	R12
	56.00
	Mineral
	8.00

	
	CAVALIER
	

	2003-2004
	All Engines - w/ CVC6 Compressor ...
	R134a
	24.00
	PAG 150
	5.00

	2002
	All Engines - w/ CVC6 Compressor ...
	R134a
	24.00
	PAG 150
	5.00

	
	All Engines - w/ V5 Compressor ...
	R134a
	24.00
	PAG 150
	9.00

	1995-2001
	All Engines
	R134a
	24.00
	PAG 150
	9.00

	1994
	All Engines
	R134a
	36.00
	PAG 150
	8.00

	1986-1993
	All Engines
	R12
	36.00
	Mineral
	8.00

	1985
	All Engines
	R12
	40.00
	Mineral
	7.00

	
	CELEBRITY
	

	1985-1990
	All Engines
	R12
	44.00
	Mineral
	8.00

	
	CORSICA
	

	1994-1996
	All Engines
	R134a
	36.00
	PAG 150
	9.00

	1992-1993
	All Engines
	R12
	42.00
	Mineral
	8.00

	1991
	All Engines
	R12
	36.00
	Mineral
	8.00

	1988-1990
	All Engines
	R12
	44.00
	Mineral
	8.00

	
	CORVETTE
	

	2002-2004
	All Engines
	R134a
	28.00
	PAG 150
	9.00

	2000-2001
	All Engines
	R134a
	24.00
	PAG 150
	9.00

	1997-1999
	All Engines
	R134a
	26.00
	PAG 150
	9.00

	1994-1996
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1990-1993
	All Engines
	R12
	36.00
	Mineral
	8.00

	1989
	All Engines
	R12
	44.00
	Mineral
	8.00

	1985
	All Engines
	R12
	52.00
	Mineral
	7.00

	
	IMPALA
	

	2004
	All Engines
	R134a
	37.00
	PAG 150
	8.00

	2000-2003
	All Engines
	R134a
	36.00
	PAG 150
	9.00

	
	IMPALA SS
	

	1994-1996
	All Engines
	R134a
	28.00
	PAG 150
	8.00

	
	LUMINA
	

	1998-2001
	All Engines
	R134a
	36.00
	PAG 150
	9.00

	1997
	All Engines
	R134a
	36.00
	PAG 150
	8.00

	1994-1996
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1993
	All Engines
	R12
	36.00
	Mineral
	9.00

	1990-1992
	All Engines
	R12
	36.00
	Mineral
	8.00

	
	LUMINA APV
	

	1994-1996
	All Engines w/ Front AC Only ...
	R134a
	36.00
	PAG 150
	8.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	48.00
	PAG 150
	11.00

	1993
	All Engines w/ Front AC Only ...
	R134a
	42.00
	PAG 150
	8.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	56.00
	PAG 150
	11.00

	1992
	All Engines w/ Front AC Only ...
	R12
	51.00
	Mineral
	8.00

	
	All Engines w/ Front & Rear AC ...
	R12
	67.00
	Mineral
	8.00

	1991
	All Engines w/ Front AC Only ...
	R12
	51.00
	Mineral
	8.00

	1990
	All Engines w/ Front AC Only ...
	R12
	44.00
	Mineral
	8.00

	
	MALIBU
	

	2004
	All Engines
	R134a
	18.00
	PAG 150
	5.00

	2002-2003
	All Engines
	R134a
	22.00
	PAG 150
	9.00

	1997-2001
	All Engines
	R134a
	28.00
	PAG 150
	9.00

	
	METRO
	

	2001
	All Engines
	R134a
	21.00
	PAG 150
	5.00

	1998-1999
	All Engines
	R134a
	21.00
	PAG 150
	3.50

	
	MONTE CARLO
	

	2004
	All Engines
	R134a
	37.00
	PAG 150
	8.00

	1998-2003
	All Engines
	R134a
	36.00
	PAG 150
	9.00

	1997
	All Engines
	R134a
	36.00
	PAG 150
	8.00

	1995-1996
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1986-1987
	All Engines
	R12
	56.00
	Mineral
	7.00

	1985
	All Engines
	R12
	52.00
	Mineral
	7.00

	
	All Engines
	R12
	52.00
	Mineral
	7.00

	
	NOVA
	

	1987-1988
	All Engines
	R12
	26.00
	Mineral
	7.00

	1986
	All Engines
	R12
	27.00
	Mineral
	7.00

	
	PRIZM
	

	1998-2002
	All Engines
	R134a
	26.50
	PAG 150
	8.00

	
	VENTURE
	

	2004
	All Engines w/ Front & Rear AC ...
	R134a
	35.00
	PAG 46
	8.00

	
	All Engines w/ Front AC Only ...
	R134a
	27.00
	PAG 46
	7.00

	2003
	All Engines w/ Front & Rear AC ...
	R134a
	35.00
	PAG 46
	8.00

	2002
	All Engines w/ Front & Rear AC ...
	R134a
	35.00
	PAG 46
	8.00

	
	All Engines w/ Front AC Only ...
	R134a
	27.00
	PAG 46
	7.00

	
	All Engines w/ Front AC Only ...
	R134a
	27.00
	PAG 46
	7.00

	2001
	All Engines w/ Front AC Only ...
	R134a
	27.00
	PAG 46
	7.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	35.00
	PAG 46
	8.00

	1997-2000
	All Engines w/ Front AC Only ...
	R134a
	32.00
	PAG 150
	8.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	46.00
	PAG 150
	11.00


Chevrolet Light Truck
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	ASTRO
	

	1994-2004
	All Engines w/ Front & Rear AC ...
	R134a
	48.00
	PAG 150
	11.00

	
	All Engines w/ Front AC Only ...
	R134a
	32.00
	PAG 150
	8.00

	1992-1993
	All Engines w/ Front & Rear AC ...
	R12
	60.00
	Mineral
	10.00

	
	All Engines w/ Front AC Only ...
	R12
	48.00
	Mineral
	8.00

	1990-1991
	All Engines w/ Front & Rear AC ...
	R12
	60.00
	Mineral
	10.00

	
	All Engines w/ Front AC Only ...
	R12
	36.00
	Mineral
	8.00

	1989
	All Engines w/ Front & Rear AC ...
	R12
	60.00
	Mineral
	10.00

	
	All Engines w/ Front AC Only ...
	R12
	32.00
	Mineral
	8.00

	1985-1988
	All Engines w/ Front AC Only ...
	R12
	32.00
	Mineral
	8.00

	
	All Engines w/ Front & Rear AC ...
	R12
	48.00
	Mineral
	9.00

	
	AVALANCHE
	

	2003-2004
	All Engines - w/ Rear AC ...
	R134a
	43.00
	PAG 46
	11.00

	
	All Engines - w/o Rear AC ...
	R134a
	25.50
	PAG 46
	8.00

	2002
	All Engines - w/ Rear AC ...
	R134a
	42.00
	PAG 46
	11.00

	
	All Engines - w/o Rear AC ...
	R134a
	28.00
	PAG 46
	8.00

	
	FULLSIZE PICKUP
	

	2003-2004
	All Engines - w/ Denso Compressor ...
	R134a
	25.50
	PAG 46
	8.00

	2002
	All Engines - w/ Denso Compressor ...
	R134a
	29.00
	PAG 46
	8.00

	2000-2001
	All Engines - w/ Denso Compressor ...
	R134a
	29.00
	PAG 46
	8.00

	
	All Engines - w/ HT6 Compressor ...
	R134a
	32.00
	PAG 150
	8.00

	1994-1999
	All Engines - Crew Cab ...
	R134a
	36.00
	PAG 150
	8.00

	
	All Engines - Standard Cab ...
	R134a
	32.00
	PAG 150
	8.00

	1993
	Crew Cab - w/ R12 ...
	R12
	48.00
	Mineral
	8.00

	
	Standard Cab - w/ R12 ...
	R12
	40.00
	Mineral
	8.00

	
	Crew Cab - w/ R134a ...
	R134a
	48.00
	PAG 150
	8.00

	
	Standard Cab - w/ R134a ...
	R134a
	40.00
	PAG 150
	8.00

	1992
	All Engines
	R12
	44.00
	Mineral
	8.00

	1990-1991
	C,K ...
	R12
	44.00
	Mineral
	8.00

	
	R,V ...
	R12
	52.00
	Mineral
	8.00

	1989
	C,K ...
	R12
	40.00
	Mineral
	8.00

	
	R,V ...
	R12
	52.00
	Mineral
	8.00

	1988
	C,K ...
	R12
	40.00
	Mineral
	8.00

	1987
	All Engines
	R12
	48.00
	Mineral
	8.00

	1985-1986
	All Engines - w/ A6 Compressor ...
	R12
	44.00
	Mineral
	10.00

	
	All Engines - w/ R4 Compressor ...
	R12
	44.00
	Mineral
	8.00

	
	G SERIES FULLSIZE VAN / EXPRESS
	

	2003-2004
	All Engines w/ Front & Rear AC ...
	R134a
	48.00
	PAG 46
	11.00

	
	All Engines w/ Front AC Only ...
	R134a
	32.00
	PAG 46
	8.00

	2002
	All Engines w/ Front AC Only ...
	R134a
	32.00
	PAG 150
	8.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	56.00
	PAG 150
	11.00

	2000-2001
	All Engines w/ Front AC Only ...
	R134a
	48.00
	PAG 150
	8.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	72.00
	PAG 150
	11.00

	1997-1999
	All Engines w/ Front AC Only ...
	R134a
	48.00
	PAG 150
	8.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	78.00
	PAG 150
	11.00

	1996
	All Engines w/ Front AC Only ...
	R134a
	36.00
	PAG 150
	8.00

	1996
	All Engines w/ Front & Rear AC ...
	R134a
	78.00
	PAG 150
	11.00

	1994-1995
	All Engines w/ Front AC Only ...
	R134a
	48.00
	PAG 150
	8.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	68.00
	PAG 150
	11.00

	1993
	All Engines w/ Front & Rear AC ...
	R12
	68.00
	Mineral
	11.00

	
	All Engines w/ Front AC Only ...
	R12
	48.00
	Mineral
	8.00

	1992
	All Engines w/ Front & Rear AC ...
	R12
	60.00
	Mineral
	11.00

	
	All Engines w/ Front AC Only ...
	R12
	48.00
	Mineral
	8.00

	1991
	Exc. 7.4 Eng. w/ Front & Rear AC ...
	R12
	72.00
	Mineral
	11.00

	
	Exc. 7.4 Eng. w/ Front AC ...
	R12
	48.00
	Mineral
	8.00

	
	8 Cyl. 7.4 Eng. w/ Front & Rear AC ...
	R12
	64.00
	Mineral
	11.00

	
	8 Cyl. 7.4 Eng. w/ Front AC Only ...
	R12
	40.00
	Mineral
	8.00

	1990
	Exc. 7.4 Eng. w/ Front AC - w/ A6 Compressor ...
	R12
	56.00
	Mineral
	10.00

	
	Exc. 7.4 Eng. w/ Front & Rear AC - w/ A6 Compressor ...
	R12
	72.00
	Mineral
	13.00

	
	Exc. 7.4 Eng. w/ Front & Rear AC - w/ R4 Compressor ...
	R12
	72.00
	Mineral
	9.00

	
	Exc. 7.4 Eng. w/ Front AC - w/ R4 Compressor ...
	R12
	56.00
	Mineral
	8.00

	
	8 Cyl. 7.4 Eng. w/ Front AC Only - w/ A6 Compressor ...
	R12
	46.00
	Mineral
	10.00

	
	8 Cyl. 7.4 Eng. w/ Front & Rear AC - w/ A6 Compressor ...
	R12
	64.00
	Mineral
	13.00

	
	8 Cyl. 7.4 Eng. w/ Front & Rear AC - w/ R4 Compressor ...
	R12
	64.00
	Mineral
	9.00

	
	8 Cyl. 7.4 Eng. w/ Front AC Only - w/ R4 Compressor ...
	R12
	46.00
	Mineral
	8.00

	1989
	All Engines Gas w/ Front & Rear AC - w/ A6 Compressor ...
	R12
	72.00
	Mineral
	13.00

	
	All Engines Gas w/ Front AC Only - w/ A6 Compressor ...
	R12
	56.00
	Mineral
	10.00

	
	All Engines Diesel w/ Front & Rear AC - w/ A6 Compressor ...
	R12
	64.00
	Mineral
	13.00

	
	All Engines Diesel w/ Front AC Only - w/ A6 Compressor ...
	R12
	46.00
	Mineral
	10.00

	
	All Engines Gas w/ Front AC Only - w/ R4 Compressor ...
	R12
	56.00
	Mineral
	8.00

	
	All Engines Gas w/ Front & Rear AC - w/ R4 Compressor ...
	R12
	72.00
	Mineral
	9.00

	
	All Engines Diesel w/ Front AC Only - w/ R4 Compressor ...
	R12
	46.00
	Mineral
	8.00

	
	All Engines Diesel w/ Front & Rear AC - w/ R4 Compressor ...
	R12
	64.00
	Mineral
	9.00

	1988
	All Engines w/ Front AC Only ...
	R12
	56.00
	Mineral
	8.00

	
	All Engines w/ Front & Rear AC ...
	R12
	72.00
	Mineral
	11.00

	1985-1987
	All Engines w/ Front AC Only ...
	R12
	48.00
	Mineral
	8.00

	
	All Engines w/ Front & Rear AC ...
	R12
	72.00
	Mineral
	11.00

	
	S10 BLAZER
	

	2001-2004
	All Engines
	R134a
	28.00
	PAG 150
	8.00

	1999-2000
	All Engines
	R134a
	31.00
	PAG 150
	8.50

	1995-1998
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1994
	All Engines
	R134a
	40.00
	PAG 150
	8.00

	1993
	All Engines
	R12
	36.00
	Mineral
	8.00

	1989-1992
	All Engines
	R12
	40.00
	Mineral
	8.00

	1988
	All Engines
	R12
	37.00
	Mineral
	8.00

	1985-1987
	All Engines
	R12
	40.00
	Mineral
	8.00

	
	S10 PICKUP
	

	1999-2004
	All Engines - w/ HT6 Compressor ...
	R134a
	30.00
	PAG 150
	8.00

	
	All Engines - w/ V7 Compressor ...
	R134a
	28.00
	PAG 150
	9.00

	1994-1998
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1990-1993
	All Engines
	R12
	40.00
	Mineral
	8.00

	1989
	All Engines
	R12
	56.00
	Mineral
	8.00

	1988
	All Engines
	R12
	37.00
	Mineral
	8.00

	1985-1987
	All Engines
	R12
	40.00
	Mineral
	8.00

	
	SSR
	

	2004
	All Engines
	R134a
	19.00
	PAG 46
	8.00

	
	SUBURBAN / FULLSIZE BLAZER / TAHOE
	

	2003-2004
	TAHOE w/ Front AC Only ...
	R134a
	25.50
	PAG 46
	8.00

	
	TAHOE w/ Front & Rear AC ...
	R134a
	43.00
	PAG 46
	11.00

	
	Suburbans Only w/ Front AC Only ...
	R134a
	25.50
	PAG 46
	8.00

	
	Suburbans Only w/ Front & Rear AC ...
	R134a
	48.00
	PAG 46
	11.00

	2002
	TAHOE w/ Front AC Only ...
	R134a
	28.00
	PAG 46
	8.00

	
	TAHOE w/ Front & Rear AC ...
	R134a
	42.00
	PAG 46
	11.00

	
	Suburbans Only w/ Front AC Only ...
	R134a
	28.00
	PAG 46
	8.00

	
	Suburbans Only w/ Front & Rear AC ...
	R134a
	48.00
	PAG 46
	11.00

	2000-2001
	Suburbans Only w/ Front & Rear AC ...
	R134a
	48.00
	PAG 46
	11.00

	
	Suburbans Only w/ Front AC Only ...
	R134a
	29.00
	PAG 46
	8.00

	
	TAHOE w/ Front AC Only ...
	R134a
	29.00
	PAG 46
	8.00

	2000-2001
	TAHOE w/ Front & Rear AC ...
	R134a
	48.00
	PAG 46
	11.00

	1994-1999
	All Engines w/ Front & Rear AC ...
	R134a
	64.00
	PAG 150
	11.00

	
	All Engines w/ Front AC Only ...
	R134a
	36.00
	PAG 150
	8.00

	1993
	All Engines w/ Front & Rear AC ...
	R12
	68.00
	Mineral
	11.00

	
	All Engines w/ Front AC Only ...
	R12
	48.00
	Mineral
	8.00

	1990-1991
	All Engines w/ Front & Rear AC ...
	R12
	84.00
	Mineral
	11.00

	
	All Engines w/ Front AC Only ...
	R12
	52.00
	Mineral
	8.00

	1989
	All Engines w/ Front & Rear AC ...
	R12
	60.00
	Mineral
	10.00

	
	All Engines w/ Front AC Only ...
	R12
	36.00
	Mineral
	8.00

	1988
	All Engines w/ Front AC Only ...
	R12
	52.00
	Mineral
	6.00

	
	All Engines w/ Front & Rear AC ...
	R12
	84.00
	Mineral
	9.00

	
	TRACKER
	

	2001-2004
	All Engines
	R134a
	16.00
	PAG 46
	3.50

	1999-2000
	All Engines
	R134a
	16.00
	PAG 150
	3.50

	1994-1998
	All Engines
	R134a
	21.00
	PAG 150
	3.50

	1989-1993
	All Engines
	R12
	21.00
	Mineral
	5.00

	
	TRAILBLAZER
	

	2002-2004
	All Engines w/ Front & Rear AC ...
	R134a
	48.00
	PAG 46
	8.50

	
	All Engines w/ Front AC Only ...
	R134a
	29.00
	PAG 46
	7.50


Chrysler
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	300 SERIES
	

	1999-2003
	300M ...
	R134a
	25.00
	PAG 46
	5.00

	
	CIRRUS
	

	2000
	All Engines
	R134a
	19.00
	PAG 46
	5.00

	1999
	All Engines
	R134a
	20.00
	PAG 46
	5.00

	1998
	All Engines
	R134a
	26.00
	PAG 46
	5.00

	1995-1997
	All Engines
	R134a
	28.00
	PAG 46
	5.00

	
	CONCORDE
	

	1999-2003
	All Engines
	R134a
	25.00
	PAG 46
	5.00

	1997-1998
	All Engines
	R134a
	28.00
	PAG 46
	5.00

	1996
	All Engines
	R134a
	28.00
	PAG 46
	7.00

	1993-1995
	All Engines
	R134a
	28.00
	PAG 46
	4.75

	
	CONQUEST
	

	1989
	All Engines
	R12
	29.00
	Mineral
	5.75

	
	CROSSFIRE
	

	2004
	All Engines
	R134a
	32.00
	PAG 46
	4.50

	
	FIFTH AVENUE
	

	1988
	All Engines
	R12
	41.00
	Mineral
	7.25

	1985-1987
	All Engines
	R12
	42.00
	Mineral
	7.00

	
	LEBARON
	

	1995
	All Engines
	R134a
	24.00
	PAG 46
	7.25

	1994
	All Engines
	R134a
	26.00
	PAG 46
	7.25

	1993
	All Engines
	R12
	32.00
	Mineral
	7.25

	1992
	All Engines
	R12
	32.00
	Mineral
	9.00

	1991
	All Engines
	R12
	32.00
	Mineral
	7.25

	1986-1990
	All Engines
	R12
	38.00
	Mineral
	7.25

	1985
	All Engines
	R12
	37.00
	Mineral
	7.25

	
	LHS
	

	1999-2001
	All Engines
	R134a
	25.00
	PAG 46
	5.00

	1997
	All Engines
	R134a
	28.00
	PAG 46
	5.00

	1996
	All Engines
	R134a
	28.00
	PAG 46
	7.00

	1994-1995
	All Engines
	R134a
	28.00
	PAG 46
	4.75

	
	NEW YORKER
	

	1996
	All Engines
	R134a
	28.00
	PAG 46
	7.00

	1994-1995
	All Engines
	R134a
	28.00
	PAG 46
	4.75

	1991-1993
	All Engines
	R12
	32.00
	Mineral
	7.25

	1989-1990
	All Engines - w/ Fixed Displacement Compressor ...
	R12
	38.00
	Mineral
	7.25

	
	All Engines - w/ Variable Displacement Compressor ...
	R12
	38.00
	Mineral
	8.75

	1988
	All Engines
	R12
	38.00
	Mineral
	7.25

	
	NEW YORKER / FIFTH AVENUE
	

	1991
	All Engines
	R12
	34.00
	Mineral
	9.00

	1990
	All Engines
	R12
	38.00
	Mineral
	8.75

	
	PT CRUISER
	

	2002-2004
	All Engines
	R134a
	25.00
	PAG 46
	6.00

	2001
	All Engines
	R134a
	25.00
	PAG 46
	5.50

	
	SEBRING
	

	2000-2001
	CONVERTIBLE ...
	R134a
	20.00
	PAG 46
	5.00

	
	SEDAN / COUPE ...
	R134a
	26.00
	PAG 100
	6.00

	1997-1999
	CONVERTIBLE ...
	R134a
	26.00
	PAG 46
	6.50

	
	4 Cyl. SEDAN / COUPE - w/ Denso Compressor ...
	R134a
	24.00
	PAG 46
	4.00

	
	6 Cyl. SEDAN / COUPE - w/ Sanden Compressor ...
	R134a
	26.00
	PAG 100
	6.00

	1996
	CONVERTIBLE ...
	R134a
	28.00
	PAG 46
	7.00

	
	4 Cyl. SEDAN / COUPE - w/ Denso Compressor ...
	R134a
	24.00
	PAG 46
	4.00

	
	6 Cyl. SEDAN / COUPE - w/ Sanden Compressor ...
	R134a
	26.00
	PAG 100
	6.00

	1995
	4 Cyl. SEDAN / COUPE - w/ Denso Compressor ...
	R134a
	24.00
	PAG 46
	4.00

	
	6 Cyl. SEDAN / COUPE - w/ Sanden Compressor ...
	R134a
	26.00
	PAG 100
	6.00

	
	TOWN & COUNTRY
	

	2004
	All Engines w/ Front AC Only ...
	R134a
	31.00
	PAG 46
	6.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	43.00
	PAG 46
	10.00

	1996-2003
	All Engines w/ Front & Rear AC ...
	R134a
	46.00
	PAG 46
	13.00

	
	All Engines w/ Front AC Only ...
	R134a
	34.00
	PAG 46
	9.00

	1994-1995
	All Engines w/ Front & Rear AC ...
	R134a
	52.00
	PAG 46
	13.00

	
	All Engines w/ Front AC Only ...
	R134a
	36.00
	PAG 46
	7.00

	1993
	All Engines w/ Front & Rear AC ...
	R134a
	50.00
	PAG 46
	13.00

	
	All Engines w/ Front AC Only ...
	R134a
	36.00
	PAG 46
	7.00

	1992
	All Engines w/ Front & Rear AC - w/ Fixed Displacement Compressor ...
	R12
	44.00
	Mineral
	13.00

	
	All Engines w/ Front AC - w/ Fixed Displacement Compressor ...
	R12
	33.50
	Mineral
	7.25

	
	All Engines w/ Front & Rear AC - w/ Variable Displacement Compressor
	R12
	44.00
	Mineral
	13.00

	
	All Engines w/ Front AC Only - w/ Variable Displacement Compressor ...
	R12
	33.50
	Mineral
	8.75

	1990-1991
	All Engines w/ Front & Rear AC - w/ Fixed Displacement Compressor ...
	R12
	43.00
	Mineral
	13.00

	
	All Engines w/ Front AC - w/ Fixed Displacement Compressor ...
	R12
	32.00
	Mineral
	7.25

	
	All Engines w/ Front & Rear AC - w/ Variable Displacement Compressor
	R12
	43.00
	Mineral
	13.00

	
	All Engines w/ Front AC Only - w/ Variable Displacement Compressor ...
	R12
	32.00
	Mineral
	8.75


Daewoo
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	LANOS
	

	1999-2000
	All Engines
	R134a
	26.00
	PAG 100
	8.00

	
	LEGANZA
	

	1999-2000
	All Engines
	R134a
	26.00
	PAG 100
	8.00

	
	NUBIRA
	

	1999-2000
	All Engines
	R134a
	26.00
	PAG 100
	8.00


Daihatsu
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	CHARADE
	

	1989-1992
	All Engines
	R12
	23.00
	Mineral
	4.00

	
	ROCKY
	

	1990-1992
	All Engines
	R12
	26.50
	Mineral
	4.00


Dodge
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	ARIES
	

	1989
	All Engines - w/ Fixed Displacement Compressor ...
	R12
	38.00
	Mineral
	7.25

	
	All Engines - w/ Variable Displacement Compressor ...
	R12
	38.00
	Mineral
	8.75

	1987-1988
	All Engines
	R12
	38.00
	Mineral
	7.25

	1985
	All Engines
	R12
	37.00
	Mineral
	7.25

	
	AVENGER
	

	2000
	All Engines
	R134a
	26.00
	PAG 100
	6.00

	1995-1999
	4 Cyl. ..
	R134a
	26.00
	PAG 46
	4.00

	
	6 Cyl. ..
	R134a
	26.00
	PAG 100
	6.00

	
	DAYTONA
	

	1991-1993
	All Engines
	R12
	32.00
	Mineral
	7.25

	1989-1990
	All Engines
	R12
	38.00
	Mineral
	7.25

	
	DYNASTY
	

	1991-1993
	All Engines - w/ Fixed Displacement Compressor ...
	R12
	32.00
	Mineral
	7.25

	
	All Engines - w/ Variable Displacement Compressor ...
	R12
	32.00
	Mineral
	8.75

	1989-1990
	All Engines - w/ Fixed Displacement Compressor ...
	R12
	38.00
	Mineral
	7.25

	
	All Engines - w/ Variable Displacement Compressor ...
	R12
	38.00
	Mineral
	8.75

	1988
	All Engines
	R12
	38.00
	Mineral
	7.25

	
	INTREPID
	

	1999-2002
	All Engines
	R134a
	25.00
	PAG 46
	5.00

	1996-1998
	All Engines
	R134a
	28.00
	PAG 46
	5.00

	1993-1995
	All Engines
	R134a
	28.00
	PAG 46
	4.75

	
	MONACO
	

	1990-1992
	All Engines
	R12
	36.00
	Mineral
	8.00

	
	NEON
	

	2002
	All Engines
	R134a
	30.00
	PAG 46
	6.00

	2000-2001
	All Engines
	R134a
	27.00
	PAG 46
	6.00

	1999
	All Engines
	R134a
	28.00
	PAG 46
	4.75

	1997-1998
	All Engines
	R134a
	25.00
	PAG 46
	6.75

	1995-1996
	All Engines
	R134a
	29.00
	PAG 46
	4.75

	
	OMNI
	

	1988
	All Engines
	R12
	38.00
	Mineral
	7.25

	1987
	All Engines
	R12
	34.00
	Mineral
	7.25

	
	SHADOW
	

	1994
	All Engines
	R134a
	26.00
	PAG 46
	7.25

	1991-1993
	All Engines
	R12
	32.00
	Mineral
	7.25

	1987-1988
	All Engines
	R12
	38.00
	Mineral
	7.25

	
	SPIRIT
	

	1994
	All Engines
	R134a
	26.00
	PAG 46
	7.25

	1991-1993
	All Engines
	R12
	32.00
	Mineral
	7.25

	1989-1990
	All Engines
	R12
	38.00
	Mineral
	7.25

	
	STEALTH
	

	1996
	All Engines
	R134a
	30.00
	PAG 46
	5.50

	1994-1995
	All Engines
	R134a
	28.00
	PAG 46
	5.50

	1993
	All Engines
	R12
	29.00
	Mineral
	5.50

	1991-1992
	All Engines
	R12
	18.00
	Mineral
	5.50

	
	STRATUS
	

	1999-2001
	All Engines
	R134a
	20.00
	PAG 46
	5.00

	1998
	All Engines
	R134a
	26.00
	PAG 46
	5.00

	1995-1997
	All Engines
	R134a
	28.00
	PAG 46
	5.00

	
	VIPER
	

	2002
	All Engines
	R134a
	30.00
	PAG 46
	4.75

	
	All Engines
	R134a
	30.00
	PAG 46
	6.00

	1994-2001
	All Engines
	R134a
	30.00
	PAG 46
	4.75


Dodge Light Truck
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	CARAVAN / GRAND CARAVAN
	

	2004
	All Engines w/ Front & Rear AC ...
	R134a
	43.00
	PAG 46
	10.00

	
	All Engines w/ Front AC Only ...
	R134a
	31.00
	PAG 46
	6.00

	1996-2003
	All Engines w/ Front & Rear AC ...
	R134a
	46.00
	PAG 46
	13.00

	
	All Engines w/ Front AC Only ...
	R134a
	34.00
	PAG 46
	9.00

	1994-1995
	All Engines w/ Front & Rear AC ...
	R134a
	52.00
	PAG 46
	13.00

	
	All Engines w/ Front AC Only ...
	R134a
	36.00
	PAG 46
	7.00

	1993
	All Engines w/ Front & Rear AC ...
	R134a
	50.00
	PAG 46
	13.00

	
	All Engines w/ Front AC Only ...
	R134a
	36.00
	PAG 46
	7.00

	1992
	All Engines w/ Front & Rear AC - w/ Fixed Displacement Compressor ...
	R12
	44.00
	Mineral
	13.00

	
	All Engines w/ Front AC Only - w/ Fixed Displacement Compressor ...
	R12
	33.50
	Mineral
	7.25

	
	All Engines w/ Front & Rear AC - w/ Variable Displacement Compressor
	R12
	44.00
	Mineral
	13.00

	
	All Engines w/ Front AC Only - w/ Variable Displacement Compressor ...
	R12
	33.50
	Mineral
	8.75

	1990-1991
	All Engines w/ Front & Rear AC - w/ Fixed Displacement Compressor ...
	R12
	43.00
	Mineral
	13.00

	
	All Engines w/ Front AC Only - w/ Fixed Displacement Compressor ...
	R12
	32.00
	Mineral
	7.25

	
	All Engines w/ Front & Rear AC - w/ Variable Displacement Compressor
	R12
	43.00
	Mineral
	13.00

	
	All Engines w/ Front AC Only - w/ Variable Displacement Compressor ...
	R12
	32.00
	Mineral
	8.75

	1989
	All Engines w/ Front & Rear AC - w/ Fixed Displacement Compressor ...
	R12
	43.00
	Mineral
	13.00

	
	All Engines w/ Front AC Only - w/ Fixed Displacement Compressor ...
	R12
	38.00
	Mineral
	7.25

	
	All Engines w/ Front & Rear AC - w/ Variable Displacement Compressor
	R12
	43.00
	Mineral
	13.00

	
	All Engines w/ Front AC Only - w/ Variable Displacement Compressor ...
	R12
	38.00
	Mineral
	8.75

	1988
	All Engines w/ Front AC Only ...
	R12
	44.00
	Mineral
	7.25

	1986-1987
	All Engines w/ Front AC Only ...
	R12
	38.00
	Mineral
	7.00

	
	DAKOTA
	

	2001-2004
	All Engines
	R134a
	32.00
	PAG 100
	7.00

	2000
	All Engines
	R134a
	32.00
	PAG 100
	6.00

	1998-1999
	All Engines
	R134a
	32.00
	PAG 100
	8.00

	1994-1997
	All Engines
	R134a
	32.00
	PAG 100
	7.75

	1992-1993
	All Engines
	R12
	40.00
	Mineral
	7.25

	1987-1991
	All Engines
	R12
	44.00
	Mineral
	7.25

	
	DURANGO
	

	2001-2004
	All Engines w/ Front AC Only ...
	R134a
	--
	PAG 100
	7.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	--
	PAG 100
	10.00

	2000
	All Engines w/ Front & Rear AC ...
	R134a
	32.00
	PAG 100
	8.00

	
	All Engines w/ Front AC Only ...
	R134a
	28.00
	PAG 100
	5.50

	1998-1999
	All Engines w/ Front & Rear AC ...
	R134a
	30.00
	PAG 100
	8.00

	
	All Engines w/ Front AC Only ...
	R134a
	28.00
	PAG 100
	5.50

	
	PICKUP - FULLSIZE / RAMCHARGER
	

	2003
	All Engines
	R134a
	32.00
	PAG 100
	6.25

	2002
	All Engines
	R134a
	26.00
	PAG 46
	6.25

	2000-2001
	All Engines
	R134a
	32.00
	PAG 100
	6.25

	1998-1999
	All Engines
	R134a
	32.00
	PAG 100
	8.00

	1994-1997
	All Engines
	R134a
	32.00
	PAG 100
	7.75

	1993
	All Engines
	R12
	44.00
	Mineral
	4.75

	1988-1992
	All Engines
	R12
	44.00
	Mineral
	7.25

	1985-1987
	All Engines
	R12
	42.00
	Mineral
	7.25

	
	PICKUP - MINI
	

	1990-1993
	All Engines
	R12
	30.00
	Mineral
	8.50

	1989
	All Engines
	R12
	32.00
	Mineral
	6.00

	
	RAIDER
	

	1989
	All Engines
	R12
	32.00
	Mineral
	6.75

	
	VAN - FULLSIZE
	

	1998-2003
	All Engines w/ Front & Rear AC ...
	R134a
	46.00
	PAG 46
	10.00

	
	All Engines w/ Front AC Only ...
	R134a
	34.00
	PAG 46
	8.00

	1995-1997
	All Engines w/ Front & Rear AC ...
	R134a
	60.00
	PAG 46
	9.00

	
	All Engines w/ Front AC Only ...
	R134a
	40.00
	PAG 46
	7.25

	1994
	All Engines w/ Front & Rear AC ...
	R12
	60.00
	Mineral
	9.00

	
	All Engines w/ Front AC Only ...
	R12
	40.00
	Mineral
	7.25

	1991-1993
	All Engines w/ Front & Rear AC ...
	R12
	65.00
	Mineral
	9.00

	
	All Engines w/ Front AC Only ...
	R12
	44.00
	Mineral
	7.25

	1990
	All Engines w/ Front AC Only ...
	R12
	44.00
	Mineral
	7.25

	
	All Engines w/ Front & Rear AC - w/ Fixed Displacement Compressor ...
	R12
	65.00
	Mineral
	9.00

	
	All Engines w/ Front & Rear AC - w/ Variable Displacement Compressor
	R12
	65.00
	Mineral
	9.00

	1989
	All Engines w/ Front & Rear AC - w/ Fixed Displacement Compressor ...
	R12
	65.00
	Mineral
	9.00

	
	All Engines w/ Front AC Only - w/ Fixed Displacement Compressor ...
	R12
	49.00
	Mineral
	7.25

	
	All Engines w/ Front & Rear AC - w/ Variable Displacement Compressor
	R12
	65.00
	Mineral
	9.00

	1988
	All Engines w/ Front AC Only ...
	R12
	44.00
	Mineral
	5.00

	
	All Engines w/ Front & Rear AC ...
	R12
	65.00
	Mineral
	8.00

	1985-1987
	All Engines w/ Front AC Only ...
	R12
	48.00
	Mineral
	5.00

	
	All Engines w/ Front & Rear AC ...
	R12
	64.00
	Mineral
	8.00


Eagle
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	MEDALLION
	

	1989
	All Engines
	R12
	29.00
	Mineral
	5.50

	
	PREMIER
	

	1989-1992
	All Engines
	R12
	36.00
	Mineral
	8.00

	
	TALON
	

	1995-1998
	All Engines
	R134a
	25.50
	PAG 100
	6.50

	1990-1994
	All Engines
	R12
	33.00
	Mineral
	5.00

	
	VISION
	

	1996-1997
	All Engines
	R134a
	28.00
	PAG 46
	7.00

	1993-1995
	All Engines
	R134a
	28.00
	PAG 46
	5.00


Ford
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	ASPIRE
	

	1997
	All Engines
	R134a
	26.00
	PAG 46
	7.00

	1994-1996
	All Engines
	R134a
	25.00
	PAG 46
	6.00

	
	CONTOUR
	

	1995-2000
	All Engines
	R134a
	26.00
	PAG 46
	7.00

	
	CROWN VICTORIA
	

	1997-2004
	All Engines
	R134a
	38.00
	PAG 46
	7.50

	1995-1996
	All Engines
	R134a
	38.00
	PAG 46
	7.00

	1994
	All Engines
	R134a
	34.00
	PAG 46
	7.00

	1993
	All Engines
	R12
	38.00
	Mineral
	7.00

	1992
	All Engines
	R12
	36.00
	Mineral
	7.00

	1989-1991
	All Engines
	R12
	48.00
	Mineral
	8.00

	1988
	All Engines
	R12
	52.00
	Mineral
	13.00

	1987
	All Engines To 9/86 ...
	R12
	52.00
	Mineral
	10.00

	
	All Engines From 9/86 ...
	R12
	52.00
	Mineral
	13.00

	
	ESCORT
	

	1997-2002
	All Engines
	R134a
	28.00
	PAG 46
	6.75

	1994-1996
	All Engines
	R134a
	28.00
	PAG 46
	8.00

	1993
	All Engines
	R12
	34.08
	Mineral
	7.75

	1991-1992
	All Engines
	R12
	33.92
	Mineral
	8.00

	1988-1990
	All Engines
	R12
	36.00
	Mineral
	8.00

	1987
	All Engines
	R12
	41.00
	Mineral
	8.00

	1986
	All Engines
	R12
	36.00
	Mineral
	10.00

	
	FESTIVA
	

	1988-1993
	All Engines
	R12
	25.00
	Mineral
	10.00

	
	FOCUS
	

	2000-2004
	All Engines
	R134a
	26.00
	PAG 46
	7.00

	
	LTD
	

	1986
	All Engines
	R12
	40.00
	Mineral
	8.00

	1985
	All Engines
	R12
	40.00
	Mineral
	8.00

	
	All Engines
	R12
	40.00
	Mineral
	10.00

	
	MUSTANG
	

	1997-2004
	All Engines
	R134a
	34.00
	PAG 46
	8.50

	1994-1996
	All Engines
	R134a
	34.00
	PAG 46
	7.00

	1989-1993
	All Engines
	R12
	40.00
	Mineral
	8.00

	1986-1988
	All Engines
	R12
	40.00
	Mineral
	10.00

	
	PROBE
	

	1997
	All Engines
	R134a
	28.00
	PAG 100
	7.00

	1994-1996
	All Engines
	R134a
	28.00
	PAG 100
	6.75

	1993
	All Engines
	R12
	28.00
	Mineral
	8.00

	1989-1992
	All Engines
	R12
	40.00
	Mineral
	8.00

	
	TAURUS
	

	1996-2004
	All Engines
	R134a
	34.00
	PAG 46
	7.00

	1995
	All Engines
	R134a
	32.00
	PAG 46
	7.00

	1994
	Excl. SHO ...
	R134a
	33.00
	PAG 46
	7.00

	
	SHO ...
	R134a
	33.00
	PAG 46
	8.00

	1993
	SHO ...
	R12
	32.00
	Mineral
	8.00

	
	Excl. SHO ...
	R12
	32.00
	Mineral
	7.00

	1992
	Excl. SHO ...
	R12
	41.00
	Mineral
	7.00

	
	SHO ...
	R12
	41.00
	Mineral
	8.00

	1991
	4 Cyl. 2.5 Eng. ..
	R12
	41.00
	Mineral
	10.00

	
	6 Cyl. Excl. SHO ...
	R12
	41.00
	Mineral
	7.00

	
	6 Cyl. SHO ...
	R12
	41.00
	Mineral
	8.00

	1990
	4 Cyl. 2.5 Eng. ..
	R12
	41.00
	Mineral
	10.00

	
	6 Cyl. Excl. SHO ...
	R12
	41.00
	Mineral
	8.00

	
	6 Cyl. SHO ...
	R12
	41.00
	Mineral
	8.00

	1989
	4 Cyl. 2.5 Eng. ..
	R12
	44.00
	Mineral
	10.00

	
	6 Cyl. Excl. SHO ...
	R12
	40.00
	Mineral
	8.00

	
	6 Cyl. SHO ...
	R12
	44.00
	Mineral
	8.00

	1988
	6 Cyl. 3.0 Eng. ..
	R12
	44.00
	Mineral
	8.00

	
	6 Cyl. 3.8 Eng. ..
	R12
	40.00
	Mineral
	8.00

	1987
	4 Cyl. 2.5 Eng. ..
	R12
	44.00
	Mineral
	10.00

	
	6 Cyl. 3.0 Eng. ..
	R12
	44.00
	Mineral
	8.00

	1986
	All Engines
	R12
	44.00
	Mineral
	10.00

	
	TEMPO
	

	1994
	4 Cyl. 2.3 Eng. ..
	R134a
	33.00
	PAG 46
	7.00

	
	6 Cyl. 3.0 Eng. ..
	R134a
	33.00
	PAG 46
	7.00

	1992-1993
	4 Cyl. 2.3 Eng. ..
	R12
	36.00
	Mineral
	7.00

	
	6 Cyl. 3.0 Eng. ..
	R12
	36.00
	Mineral
	8.00

	1989-1991
	All Engines
	R12
	36.00
	Mineral
	7.00

	1987-1988
	All Engines
	R12
	36.00
	Mineral
	8.00

	1986
	All Engines
	R12
	36.00
	Mineral
	10.00

	1985
	All Engines
	R12
	41.00
	Mineral
	10.00

	
	THUNDERBIRD
	

	2002-2004
	All Engines
	R134a
	28.00
	PAG 46
	7.00

	1995-1997
	All Engines
	R134a
	36.00
	PAG 46
	7.00

	1994
	All Engines
	R134a
	36.00
	PAG 46
	6.75

	1989-1993
	All Engines
	R12
	40.00
	Mineral
	7.00

	1988
	4 Cyl. ..
	R12
	40.00
	Mineral
	8.00

	
	6 Cyl. ..
	R12
	40.00
	Mineral
	8.00

	
	8 Cyl. ..
	R12
	40.00
	Mineral
	10.00

	1987
	All Engines
	R12
	40.00
	Mineral
	8.00

	1986
	All Engines To 1/86 ...
	R12
	40.00
	Mineral
	10.00

	
	All Engines From 1/86 ...
	R12
	40.00
	Mineral
	8.00


Ford Light Truck
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	AEROSTAR
	

	1997
	All Engines w/ Front & Rear AC ...
	R134a
	54.00
	PAG 46
	13.00

	
	All Engines w/ Front AC Only ...
	R134a
	38.00
	PAG 46
	9.00

	1994-1996
	All Engines w/ Front & Rear AC ...
	R134a
	54.00
	PAG 46
	10.00

	
	All Engines w/ Front AC Only ...
	R134a
	38.00
	PAG 46
	7.00

	1993
	All Engines w/ Front & Rear AC ...
	R12
	64.00
	Mineral
	10.00

	
	All Engines w/ Front AC Only ...
	R12
	46.00
	Mineral
	7.00

	1989-1992
	All Engines w/ Front & Rear AC ...
	R12
	68.00
	Mineral
	10.00

	
	All Engines w/ Front AC Only ...
	R12
	56.00
	Mineral
	7.00

	1988
	All Engines w/ Front AC Only - w/ 6E171 Compressor ...
	R12
	56.00
	Mineral
	13.00

	
	All Engines w/ Front & Rear AC - w/ 6E171 Compressor ...
	R12
	60.00
	Mineral
	13.00

	
	All Engines w/ Front AC Only - w/ FS6 Compressor ...
	R12
	56.00
	Mineral
	10.00

	
	All Engines w/ Front & Rear AC - w/ FS6 Compressor ...
	R12
	60.00
	Mineral
	10.00

	1987
	All Engines w/ Front AC Only To 9/22/86 ...
	R12
	56.00
	Mineral
	10.00

	
	All Engines w/ Front AC Only From 9/22/86 ...
	R12
	56.00
	Mineral
	13.00

	
	All Engines w/ Front & Rear AC To 9/22/86 ...
	R12
	60.00
	Mineral
	10.00

	
	All Engines w/ Front & Rear AC From 9/22/86 ...
	R12
	60.00
	Mineral
	13.00

	
	BRONCO
	

	1996
	All Engines
	R134a
	38.00
	PAG 46
	8.00

	1995
	All Engines
	R134a
	33.00
	PAG 46
	11.00

	1994
	All Engines
	R134a
	32.00
	PAG 46
	7.00

	1991-1993
	All Engines
	R12
	44.00
	Mineral
	7.00

	1989-1990
	All Engines
	R12
	52.00
	Mineral
	7.00

	
	BRONCO II
	

	1990
	All Engines
	R12
	32.00
	Mineral
	10.00

	1988-1989
	All Engines
	R12
	40.00
	Mineral
	10.00

	1987
	All Engines
	R12
	44.00
	Mineral
	10.00

	1986
	All Engines
	R12
	52.00
	Mineral
	10.00

	1985
	All Engines
	R12
	48.00
	Mineral
	10.00

	
	E SERIES FULLSIZE VAN
	

	2002-2004
	All Engines w/ Front AC ...
	R134a
	40.00
	PAG 46
	9.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	60.00
	PAG 46
	13.00

	1997-2001
	All Engines w/ Front AC ...
	R134a
	44.00
	PAG 46
	9.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	64.00
	PAG 46
	13.00

	1995-1996
	All Engines w/ Front AC ...
	R134a
	44.00
	PAG 46
	7.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	64.00
	PAG 46
	10.00

	1994
	All Engines w/ Front AC ...
	R134a
	48.00
	PAG 46
	7.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	64.00
	PAG 46
	10.00

	1992-1993
	All Engines w/ Front AC ...
	R12
	55.00
	Mineral
	10.00

	
	All Engines w/ Front & Rear AC ...
	R12
	72.00
	Mineral
	10.00

	1989-1991
	All Engines w/ Front AC ...
	R12
	56.00
	Mineral
	10.00

	
	All Engines w/ Front & Rear AC ...
	R12
	68.00
	Mineral
	10.00

	1985-1988
	All Engines w/ Single Unit ...
	R12
	56.00
	Mineral
	10.00

	
	All Engines w/ Dual Unit ...
	R12
	68.00
	Mineral
	13.00

	
	ESCAPE
	

	2001-2004
	All Engines
	R134a
	32.00
	PAG 46
	7.00

	
	EXCURSION
	

	2001-2004
	All Engines w/ Front AC ...
	R134a
	42.00
	PAG 46
	9.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	68.00
	PAG 46
	14.00

	2000
	All Engines w/ Front AC ...
	R134a
	40.00
	PAG 46
	9.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	68.00
	PAG 46
	14.00

	
	EXPEDITION
	

	2003-2004
	All Engines w/ Front & Rear AC ...
	R134a
	54.00
	PAG 46
	14.00

	
	All Engines w/ Front AC Only ...
	R134a
	32.00
	PAG 46
	9.00

	1998-2002
	All Engines w/ Front & Rear AC ...
	R134a
	62.00
	PAG 46
	11.00

	
	All Engines w/ Front AC Only ...
	R134a
	37.00
	PAG 46
	9.00

	1997
	All Engines w/ Front & Rear AC ...
	R134a
	62.00
	PAG 46
	13.00

	
	All Engines w/ Front AC Only ...
	R134a
	37.00
	PAG 46
	9.00

	
	EXPLORER
	

	2002-2004
	All Engines w/ Front & Rear AC ...
	R134a
	56.00
	PAG 46
	13.00

	
	All Engines w/ Front AC Only ...
	R134a
	34.00
	PAG 46
	9.00

	1998-2001
	All Engines
	R134a
	30.00
	PAG 46
	9.00

	1997
	All Engines
	R134a
	22.00
	PAG 46
	9.00

	1995-1996
	All Engines
	R134a
	26.00
	PAG 46
	7.00

	1994
	All Engines
	R134a
	36.00
	PAG 46
	7.00

	1991-1993
	All Engines
	R12
	32.00
	Mineral
	7.00

	
	F SERIES FULLSIZE PICKUP
	

	1998-2004
	All Engines - Lightduty ...
	R134a
	33.00
	PAG 46
	9.00

	
	All Engines - Superduty ...
	R134a
	42.00
	PAG 46
	9.00

	1996-1997
	All Engines
	R134a
	38.00
	PAG 46
	7.00

	1995
	All Engines
	R134a
	33.00
	PAG 46
	7.00

	1994
	All Engines
	R134a
	32.00
	PAG 46
	7.00

	1993
	All Engines
	R12
	44.00
	Mineral
	7.00

	1991-1992
	All Engines
	R12
	44.00
	Mineral
	10.00

	1986-1990
	All Engines
	R12
	52.00
	Mineral
	10.00

	1985
	All Engines
	R12
	48.00
	Mineral
	10.00

	
	FREESTAR
	

	2004
	All Engines w/ Front & Rear AC ...
	R134a
	53.00
	PAG 46
	13.00

	
	All Engines w/ Front AC Only ...
	R134a
	36.00
	PAG 46
	10.00

	
	RANGER
	

	1998-2004
	All Engines
	R134a
	30.00
	PAG 46
	9.00

	1995-1997
	All Engines
	R134a
	22.00
	PAG 46
	7.00

	1994
	All Engines
	R134a
	36.00
	PAG 46
	7.00

	1993
	All Engines
	R12
	32.00
	Mineral
	7.00

	1991-1992
	4 Cyl. 2.3 Eng. ..
	R12
	36.00
	Mineral
	7.00

	
	6 Cyl. 2.9 Eng. ..
	R12
	32.00
	Mineral
	10.00

	
	6 Cyl. 3.0 Eng. ..
	R12
	36.00
	Mineral
	7.00

	
	6 Cyl. 4.0 Eng. ..
	R12
	36.00
	Mineral
	7.00

	1990
	All Engines
	R12
	32.00
	Mineral
	10.00

	1988-1989
	All Engines
	R12
	40.00
	Mineral
	10.00

	1987
	All Engines
	R12
	44.00
	Mineral
	10.00

	1985-1986
	All Engines
	R12
	40.00
	Mineral
	10.00

	
	WINDSTAR
	

	2001-2003
	All Engines w/ Front & Rear AC ...
	R134a
	56.00
	PAG 46
	14.00

	
	All Engines w/ Front AC Only ...
	R134a
	44.00
	PAG 46
	10.00

	1999-2000
	All Engines w/ Front & Rear AC ...
	R134a
	56.00
	PAG 46
	11.00

	
	All Engines w/ Front AC Only ...
	R134a
	44.00
	PAG 46
	7.00

	1998
	All Engines w/ Front & Rear AC ...
	R134a
	56.00
	PAG 46
	11.00

	
	All Engines w/ Front AC Only ...
	R134a
	44.00
	PAG 46
	10.00

	1997
	All Engines w/ Front & Rear AC ...
	R134a
	56.00
	PAG 46
	14.00

	
	All Engines w/ Front AC Only ...
	R134a
	44.00
	PAG 46
	10.00

	1995-1996
	All Engines w/ Front & Rear AC ...
	R134a
	56.00
	PAG 46
	13.00

	1995-1996
	All Engines w/ Front AC Only ...
	R134a
	44.00
	PAG 46
	9.00


Geo
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	METRO
	

	1997
	All Engines
	R134a
	21.00
	PAG 150
	3.50

	1994-1996
	All Engines
	R134a
	18.00
	PAG 150
	3.50

	1990-1993
	All Engines
	R12
	18.00
	Mineral
	3.00

	
	PRIZM
	

	1997
	All Engines
	R134a
	26.50
	PAG 150
	4.00

	1995-1996
	All Engines
	R134a
	24.00
	PAG 150
	9.00

	1994
	All Engines
	R134a
	24.00
	PAG 150
	5.25

	1993
	All Engines
	R12
	24.00
	Mineral
	6.00

	1991-1992
	All Engines
	R12
	27.00
	Mineral
	6.00

	1990
	All Engines
	R12
	18.00
	Mineral
	3.00

	
	SPECTRUM
	

	1989
	All Engines
	R12
	29.00
	Mineral
	3.50

	
	STORM
	

	1992-1993
	All Engines
	R12
	21.00
	Mineral
	5.25

	1990-1991
	All Engines
	R12
	24.00
	Mineral
	5.25


GMC Light Truck
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	ENVOY
	

	2002-2004
	All Engines w/ Front & Rear AC ...
	R134a
	48.00
	PAG 46
	8.50

	
	All Engines w/ Front AC Only ...
	R134a
	29.00
	PAG 46
	7.50

	1999-2001
	All Engines
	R134a
	31.00
	PAG 150
	8.50

	1998
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	
	FULLSIZE PICKUP
	

	2003-2004
	All Engines - w/ Denso Compressor ...
	R134a
	25.50
	PAG 46
	8.00

	2002
	All Engines - w/ Denso Compressor ...
	R134a
	29.00
	PAG 46
	8.00

	2000-2001
	All Engines - w/ Denso Compressor ...
	R134a
	29.00
	PAG 46
	8.00

	
	All Engines - w/ HT6 Compressor ...
	R134a
	32.00
	PAG 150
	8.00

	1994-1999
	All Engines - Crew Cab ...
	R134a
	36.00
	PAG 150
	8.00

	
	All Engines - Standard Cab ...
	R134a
	32.00
	PAG 150
	8.00

	1993
	Standard Cab - w/ R12 ...
	R12
	40.00
	Mineral
	8.00

	
	Crew Cab - w/ R12 ...
	R12
	48.00
	Mineral
	8.00

	
	Standard Cab - w/ R134a ...
	R134a
	40.00
	PAG 150
	8.00

	
	Crew Cab - w/ R134a ...
	R134a
	48.00
	PAG 150
	8.00

	1990-1992
	All Engines
	R12
	44.00
	Mineral
	8.00

	
	All Engines
	R12
	52.00
	Mineral
	8.00

	1989
	All Engines
	R12
	40.00
	Mineral
	8.00

	
	All Engines
	R12
	52.00
	Mineral
	8.00

	1988
	All Engines
	R12
	40.00
	Mineral
	8.00

	1987
	All Engines
	R12
	48.00
	Mineral
	6.00

	1985-1986
	All Engines - w/ A6 Compressor ...
	R12
	44.00
	Mineral
	10.00

	
	All Engines - w/ R4 Compressor ...
	R12
	44.00
	Mineral
	8.00

	
	G SERIES FULLSIZE VAN / SAVANA
	

	2003-2004
	All Engines w/ Front AC Only ...
	R134a
	32.00
	PAG 46
	8.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	48.00
	PAG 46
	11.00

	2002
	All Engines w/ Front AC Only ...
	R134a
	32.00
	PAG 150
	8.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	56.00
	PAG 150
	11.00

	2000-2001
	All Engines w/ Front AC Only ...
	R134a
	48.00
	PAG 150
	8.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	72.00
	PAG 150
	11.00

	1997-1999
	All Engines w/ Front AC Only ...
	R134a
	48.00
	PAG 150
	8.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	78.00
	PAG 150
	11.00

	1996
	All Engines w/ Front AC Only ...
	R134a
	36.00
	PAG 150
	8.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	78.00
	PAG 150
	11.00

	1994-1995
	All Engines w/ Front AC Only ...
	R134a
	48.00
	PAG 150
	8.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	68.00
	PAG 150
	11.00

	1993
	All Engines w/ Front & Rear AC ...
	R12
	68.00
	Mineral
	11.00

	
	All Engines w/ Front AC ...
	R12
	48.00
	Mineral
	8.00

	1992
	All Engines w/ Front & Rear AC ...
	R12
	60.00
	Mineral
	11.00

	
	All Engines w/ Front AC ...
	R12
	48.00
	Mineral
	8.00

	1991
	Exc. 7.4 Eng. w/ Front & Rear AC ...
	R12
	72.00
	Mineral
	11.00

	
	Exc. 7.4 Eng. w/ Front AC ...
	R12
	48.00
	Mineral
	8.00

	
	8 Cyl. 7.4 Eng. w/ Front & Rear AC ...
	R12
	64.00
	Mineral
	11.00

	
	8 Cyl. 7.4 Eng. w/ Front AC ...
	R12
	40.00
	Mineral
	8.00

	1990
	Exc. 7.4 Eng. w/ Front AC - w/ A6 Compressor ...
	R12
	56.00
	Mineral
	10.00

	
	Exc. 7.4 Eng. w/ Front & Rear AC - w/ A6 Compressor ...
	R12
	72.00
	Mineral
	13.00

	
	Exc. 7.4 Eng. w/ Front & Rear AC - w/ R4 Compressor ...
	R12
	72.00
	Mineral
	9.00

	
	Exc. 7.4 Eng. w/ Front AC - w/ R4 Compressor ...
	R12
	56.00
	Mineral
	8.00

	
	8 Cyl. 7.4 Eng. w/ Front & Rear AC - w/ A6 Compressor ...
	R12
	64.00
	Mineral
	13.00

	
	8 Cyl. 7.4 Eng. w/ Front AC - w/ A6 Compressor ...
	R12
	46.00
	Mineral
	10.00

	
	8 Cyl. 7.4 Eng. w/ Front & Rear AC - w/ R4 Compressor ...
	R12
	72.00
	Mineral
	9.00

	
	8 Cyl. 7.4 Eng. w/ Front AC - w/ R4 Compressor ...
	R12
	46.00
	Mineral
	8.00

	1989
	Exc. 7.4 Eng. w/ Front & Rear AC - w/ A6 Compressor ...
	R12
	72.00
	Mineral
	13.00

	
	Exc. 7.4 Eng. w/ Front AC - w/ A6 Compressor ...
	R12
	56.00
	Mineral
	8.00

	
	Exc. 7.4 Eng. w/ Front & Rear AC - w/ R4 Compressor ...
	R12
	72.00
	Mineral
	9.00

	
	Exc. 7.4 Eng. w/ Front AC - w/ R4 Compressor ...
	R12
	56.00
	Mineral
	8.00

	
	8 Cyl. 7.4 Eng. w/ Front & Rear AC - w/ A6 Compressor ...
	R12
	64.00
	Mineral
	13.00

	
	8 Cyl. 7.4 Eng. w/ Front AC - w/ A6 Compressor ...
	R12
	46.00
	Mineral
	10.00

	
	8 Cyl. 7.4 Eng. w/ Front AC - w/ R4 Compressor ...
	R12
	46.00
	Mineral
	8.00

	
	8 Cyl. 7.4 Eng. w/ Front & Rear AC - w/ R4 Compressor ...
	R12
	64.00
	Mineral
	9.00

	1988
	All Engines w/ Front AC ...
	R12
	56.00
	Mineral
	8.00

	
	All Engines w/ Front & Rear AC ...
	R12
	72.00
	Mineral
	11.00

	1987
	All Engines w/ Front AC Only ...
	R12
	48.00
	Mineral
	8.00

	1986
	All Engines w/ Front & Rear AC ...
	R12
	72.00
	Mineral
	11.00

	
	All Engines w/ Front AC Only ...
	R12
	48.00
	Mineral
	8.00

	
	S15 JIMMY
	

	2001-2004
	All Engines
	R134a
	28.00
	PAG 150
	8.00

	1999-2000
	All Engines
	R134a
	31.00
	PAG 150
	8.50

	1995-1998
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1994
	All Engines
	R134a
	40.00
	PAG 150
	8.00

	1993
	All Engines
	R12
	36.00
	Mineral
	8.00

	1989-1992
	All Engines
	R12
	40.00
	Mineral
	8.00

	1988
	All Engines
	R12
	37.00
	Mineral
	8.00

	1986-1987
	All Engines
	R12
	40.00
	Mineral
	8.00

	
	S15 PICKUP
	

	1999-2004
	All Engines - w/ HT6 Compressor ...
	R134a
	30.00
	PAG 150
	8.00

	
	All Engines - w/ V7 Compressor ...
	R134a
	28.00
	PAG 150
	9.00

	1994-1998
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1989-1993
	All Engines
	R12
	40.00
	Mineral
	8.00

	
	SAFARI
	

	1994-2003
	All Engines w/ Front & Rear AC ...
	R134a
	48.00
	PAG 150
	11.00

	
	All Engines w/ Front AC Only ...
	R134a
	32.00
	PAG 150
	8.00

	1992-1993
	All Engines w/ Front & Rear AC ...
	R12
	60.00
	Mineral
	10.00

	
	All Engines w/ Front AC Only ...
	R12
	48.00
	Mineral
	8.00

	1990-1991
	All Engines w/ Front & Rear AC ...
	R12
	60.00
	Mineral
	10.00

	
	All Engines w/ Front AC Only ...
	R12
	36.00
	Mineral
	8.00

	1989
	All Engines w/ Front & Rear AC ...
	R12
	60.00
	Mineral
	9.00

	
	All Engines w/ Front AC Only ...
	R12
	32.00
	Mineral
	8.00

	1986-1988
	All Engines w/ Front & Rear AC ...
	R12
	48.00
	Mineral
	9.00

	
	All Engines w/ Front AC Only ...
	R12
	32.00
	Mineral
	8.00

	
	SUBURBAN / FULLSIZE JIMMY / YUKON
	

	2003-2004
	Suburbans Only w/ Front & Rear AC...
	R134a
	48.00
	PAG 46
	11.00

	
	Suburbans Only w/ Front AC Only ...
	R134a
	25.50
	PAG 46
	8.00

	
	Yukon w/ Front & Rear AC ...
	R134a
	43.00
	PAG 46
	11.00

	
	Yukon w/ Front AC Only ...
	R134a
	25.50
	PAG 46
	8.00

	2002
	Suburbans Only w/ Front & Rear AC...
	R134a
	48.00
	PAG 46
	11.00

	
	Suburbans Only w/ Front AC Only ...
	R134a
	28.00
	PAG 46
	8.00

	
	Yukon w/ Front & Rear AC ...
	R134a
	42.00
	PAG 46
	11.00

	
	Yukon w/ Front AC Only ...
	R134a
	28.00
	PAG 46
	8.00

	2000-2001
	Yukon w/ Front & Rear AC ...
	R134a
	48.00
	PAG 46
	11.00

	
	Yukon w/ Front AC Only ...
	R134a
	29.00
	PAG 46
	8.00

	
	Suburbans Only w/ Front AC Only ...
	R134a
	29.00
	PAG 46
	8.00

	
	Suburbans Only w/ Front & Rear AC...
	R134a
	48.00
	PAG 46
	11.00

	1994-1999
	All Engines w/ Front & Rear AC ...
	R134a
	64.00
	PAG 150
	11.00

	
	All Engines w/ Front AC Only ...
	R134a
	36.00
	PAG 150
	8.00

	1993
	All Engines w/ Front & Rear AC ...
	R12
	68.00
	Mineral
	11.00

	
	All Engines w/ Front AC Only ...
	R12
	48.00
	Mineral
	8.00

	1992
	All Engines w/ Front AC Only ...
	R12
	44.80
	Mineral
	6.00

	1990-1991
	All Engines w/ Front & Rear AC ...
	R12
	84.00
	Mineral
	11.00

	
	All Engines w/ Front AC Only ...
	R12
	52.00
	Mineral
	8.00

	1989
	All Engines w/ Front & Rear AC ...
	R12
	60.00
	Mineral
	10.00

	
	All Engines w/ Front AC Only ...
	R12
	36.00
	Mineral
	8.00


Honda
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	ACCORD
	

	2003-2004
	All Engines
	R134a
	19.00
	PAG 46
	5.50

	1994-2002
	All Engines
	R134a
	23.00
	PAG 46
	5.50

	1993
	All Engines
	R12
	30.00
	Mineral
	4.50

	1990-1992
	All Engines
	R12
	32.00
	Mineral
	4.00

	1989
	All Engines
	R12
	30.00
	Mineral
	5.75

	1987-1988
	All Engines
	R12
	26.00
	Mineral
	7.50

	1985-1986
	All Engines
	R12
	26.00
	Mineral
	8.00

	
	CIVIC
	

	2002-2004
	All Engines
	R134a
	18.00
	PAG 46
	5.00

	1996-2001
	All Engines
	R134a
	23.00
	PAG 46
	5.00

	1994-1995
	All Engines
	R134a
	19.00
	PAG 46
	5.00

	1993
	All Engines
	R12
	22.00
	Mineral
	5.00

	1992
	All Engines
	R12
	23.00
	Mineral
	5.00

	1991
	All Engines - w/ Matsushita Compressor ...
	R12
	33.00
	Mineral
	5.00

	
	All Engines - w/ Sanden Compressor...
	R12
	33.00
	Mineral
	4.00

	1990
	All Engines
	R12
	31.00
	Mineral
	4.50

	1989
	All Engines
	R12
	31.00
	Mineral
	5.00

	1988
	All Engines - w/ Matsushita Compressor ...
	R12
	34.00
	Mineral
	7.50

	
	All Engines - w/ Sanden Compressor...
	R12
	24.00
	Mineral
	7.50

	1987
	All Engines
	R12
	25.00
	Mineral
	7.50

	1986
	All Engines
	R12
	30.00
	Mineral
	7.50

	1985
	All Engines
	R12
	20.00
	Mineral
	7.50

	
	CIVIC DEL SOL
	

	1994-1997
	All Engines
	R134a
	19.00
	PAG 46
	4.00

	1993
	All Engines
	R12
	23.00
	Mineral
	5.00

	
	CR-V
	

	2002-2004
	All Engines
	R134a
	18.00
	PAG 46
	4.50

	1997-2001
	All Engines
	R134a
	25.00
	PAG 46
	4.50

	
	CRX
	

	1990-1991
	All Engines
	R12
	32.00
	Mineral
	4.50

	1989
	All Engines - w/ Matsushita Compressor ...
	R12
	32.00
	Mineral
	5.00

	
	All Engines - w/ Sanden Compressor...
	R12
	32.00
	Mineral
	4.00

	1987
	All Engines
	R12
	30.00
	Mineral
	7.50

	1986
	All Engines
	R12
	20.00
	Mineral
	7.50

	
	ELEMENT
	

	2003-2004
	All Engines
	R134a
	18.00
	PAG 46
	8.50

	
	INSIGHT
	

	2000-2004
	All Engines
	R134a
	19.00
	PAG 46
	4.75

	
	ODYSSEY
	

	1999-2004
	All Engines w/ Front & Rear AC ...
	R134a
	32.00
	PAG 46
	7.00

	
	All Engines w/ Front AC Only ...
	R134a
	23.00
	PAG 46
	7.00

	1995-1998
	All Engines w/ Front AC Only ...
	R134a
	23.00
	PAG 46
	5.50

	
	All Engines w/ Front & Rear AC ...
	R134a
	30.00
	PAG 46
	5.50

	
	PASSPORT
	

	1994-2002
	All Engines
	R134a
	23.00
	PAG 46
	5.00

	
	PILOT
	

	2003-2004
	All Engines
	R134a
	24.7 - 26.5
	PAG 46
	9.00

	
	PRELUDE
	

	1997-2001
	All Engines
	R134a
	26.50
	PAG 46
	4.50

	1995-1996
	All Engines
	R134a
	23.00
	PAG 46
	5.00

	1993-1994
	All Engines
	R134a
	18.00
	PAG 46
	5.00

	1992
	All Engines
	R12
	26.50
	Mineral
	5.00

	1991
	All Engines
	R12
	32.00
	Mineral
	4.00

	1990
	All Engines
	R12
	32.00
	Mineral
	5.00

	1988-1989
	All Engines
	R12
	34.00
	Mineral
	5.00

	1986-1987
	All Engines
	R12
	30.00
	Mineral
	8.00

	1985
	All Engines
	R12
	25.00
	Mineral
	8.00

	
	S2000
	

	2000-2004
	All Engines
	R134a
	22.00
	PAG 46
	5.00


Hummer
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	H2
	

	2003-2004
	All Engines
	R134a
	25.50
	PAG 46
	8.00


Hyundai
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	ACCENT
	

	2002-2004
	All Engines
	R134a
	20.00
	PAG 46
	6.00

	1996-2001
	All Engines
	R134a
	24.00
	PAG 46
	6.50

	1995
	All Engines
	R134a
	24.00
	PAG 46
	5.50

	
	ELANTRA
	

	2001-2004
	All Engines
	R134a
	24.00
	PAG 46
	5.00

	1996-2000
	All Engines
	R134a
	26.00
	PAG 46
	6.50

	1994-1995
	All Engines
	R134a
	24.00
	PAG 46
	5.00

	1992-1993
	All Engines
	R12
	33.00
	Mineral
	4.00

	
	EXCEL
	

	1994
	All Engines
	R134a
	25.00
	PAG 46
	7.75

	1993
	All Engines
	R12
	32.00
	Mineral
	8.00

	1991-1992
	All Engines
	R12
	33.00
	Mineral
	8.00

	1990
	All Engines
	R12
	30.00
	Mineral
	8.00

	1988-1989
	All Engines
	R12
	35.00
	Mineral
	5.00

	1987
	All Engines
	R12
	30.00
	Mineral
	5.00

	
	SANTA FE
	

	2001-2004
	All Engines
	R134a
	22.00
	PAG 46
	5.50

	
	SCOUPE
	

	1994-1995
	All Engines
	R134a
	25.00
	PAG 46
	5.50

	1993
	All Engines
	R12
	30.00
	Mineral
	3.00

	1991-1992
	All Engines
	R12
	33.00
	Mineral
	8.00

	
	SONATA
	

	2001-2004
	All Engines
	R134a
	25.00
	PAG 46
	5.00

	1999-2000
	All Engines
	R134a
	25.00
	PAG 46
	7.75

	1996-1998
	All Engines
	R134a
	27.00
	PAG 46
	7.75

	1995
	All Engines
	R134a
	28.00
	PAG 46
	7.75

	1989-1994
	All Engines
	R12
	33.00
	Mineral
	7.75

	
	TIBURON
	

	2001
	All Engines
	R134a
	26.00
	PAG 46
	5.00

	1999-2000
	All Engines
	R134a
	26.00
	PAG 46
	6.50

	
	XG350
	

	2003-2004
	All Engines
	R134a
	24.00
	PAG 46
	5.00


Infiniti
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	FX35
	

	2003-2004
	All Engines
	R134a
	19.00
	PAG 150
	6.00

	
	FX45
	

	2003-2004
	All Engines
	R134a
	19.00
	PAG 150
	6.00

	
	G20
	

	1999-2002
	All Engines
	R134a
	23.00
	PAG 100
	6.75

	1994-1996
	All Engines
	R134a
	28.00
	PAG 100
	6.75

	1991-1993
	All Engines
	R12
	29.00
	Mineral
	6.75

	
	G35
	

	2003-2004
	All Engines
	R134a
	19.00
	PAG 46
	6.00

	
	I30
	

	1998-2001
	All Engines
	R134a
	25.00
	PAG 46
	6.75

	1996-1997
	All Engines
	R134a
	25.00
	PAG 46
	8.50

	
	I35
	

	2002-2004
	All Engines
	R134a
	18.00
	PAG 46
	6.00

	
	J30
	

	1994-1997
	All Engines
	R134a
	28.00
	PAG 46
	8.50

	1993
	All Engines
	R134a
	26.50
	PAG 46
	8.50

	
	M30
	

	1990-1992
	All Engines
	R12
	35.00
	Mineral
	6.75

	
	M45
	

	2003-2004
	All Engines
	R134a
	21.00
	PAG 46
	6.00

	
	Q45
	

	2002-2004
	All Engines
	R134a
	21.00
	PAG 46
	6.00

	2001
	All Engines
	R134a
	23.00
	PAG 46
	6.75

	1997-2000
	All Engines
	R134a
	25.50
	PAG 46
	6.75

	1996
	All Engines
	R134a
	29.00
	PAG 46
	6.75

	1994-1995
	All Engines
	R134a
	28.00
	PAG 46
	8.50

	1993
	All Engines
	R12
	42.00
	Mineral
	10.00

	1990-1992
	All Engines
	R12
	44.00
	Mineral
	8.00

	
	QX4
	

	2002-2003
	All Engines
	R134a
	16.00
	PAG 100
	6.00

	1997-2001
	All Engines
	R134a
	25.00
	PAG 100
	6.75

	
	QX56
	

	2004
	All Engines
	R134a
	38.50
	PAG 150
	10.00


Isuzu
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	ASCENDER
	

	2003-2004
	All Engines w/ Front AC Only ...
	R134a
	29.00
	PAG 46
	7.50

	
	All Engines w/ Front & Rear AC ...
	R134a
	48.00
	PAG 46
	8.50

	
	AMIGO
	

	1999-2000
	All Engines
	R134a
	23.00
	PAG 46
	5.00

	1998
	All Engines
	R134a
	21.00
	PAG 46
	5.00

	1994
	All Engines
	R134a
	23.00
	PAG 46
	5.00

	1991-1993
	All Engines
	R12
	30.00
	Mineral
	6.00

	1989-1990
	All Engines
	R12
	30.00
	Mineral
	5.00

	
	AXIOM
	

	2002-2004
	All Engines
	R134a
	24.00
	PAG 46
	5.00

	
	HOMBRE
	

	1999-2000
	All Engines
	R134a
	30.00
	PAG 150
	8.00

	1997-1998
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1996
	All Engines
	R134a
	32.00
	PAG 150
	9.00

	
	I-MARK
	

	1989
	All Engines
	R12
	28.00
	Mineral
	6.50

	
	IMPULSE
	

	1992
	All Engines
	R12
	21.00
	Mineral
	5.00

	1989-1991
	All Engines
	R12
	35.00
	Mineral
	6.75

	
	OASIS
	

	1996-1999
	All Engines w/ Front & Rear AC ...
	R134a
	30.00
	PAG 46
	6.50

	
	All Engines w/ Front AC Only ...
	R134a
	23.00
	PAG 46
	5.50

	
	PICKUP
	

	1994-1995
	All Engines
	R134a
	23.00
	PAG 46
	5.00

	1993
	All Engines
	R12
	26.50
	Mineral
	6.00

	1989-1992
	All Engines
	R12
	30.00
	Mineral
	6.00

	
	RODEO
	

	2002-2004
	All Engines
	R134a
	24.00
	PAG 46
	5.00

	1994-2001
	All Engines
	R134a
	23.00
	PAG 46
	5.00

	1993
	All Engines
	R12
	26.50
	Mineral
	5.00

	1991-1992
	All Engines
	R12
	35.00
	Mineral
	6.00

	
	STYLUS
	

	1992-1993
	All Engines
	R12
	21.00
	Mineral
	5.00

	1991
	All Engines
	R12
	26.50
	Mineral
	6.75

	
	TROOPER
	

	1998-2001
	All Engines
	R134a
	21.00
	PAG 46
	5.00

	1994-1997
	All Engines
	R134a
	26.50
	PAG 46
	5.00

	1992-1993
	All Engines
	R12
	30.00
	Mineral
	5.00

	1989-1991
	All Engines
	R12
	33.50
	Mineral
	5.00

	1988
	All Engines
	R12
	34.00
	Mineral
	5.00

	
	VEHICROSS
	

	1999-2001
	All Engines
	R134a
	23.00
	PAG 46
	5.00


Jaguar
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	S-TYPE
	

	2003-2004
	All Engines
	R134a
	26.50
	PAG 100
	7.00

	2000-2002
	All Engines
	R134a
	28.00
	PAG 100
	7.00

	
	VANDEN PLAS
	

	1998-2000
	All Engines
	R134a
	40.00
	PAG 100
	5.00

	
	XJ12
	

	1995-1996
	All Engines
	R134a
	38.50
	PAG 100
	8.00

	1994
	All Engines
	R134a
	44.00
	PAG 100
	6.50

	
	XJ6
	

	1995-1997
	All Engines
	R134a
	38.50
	PAG 100
	7.50

	1993-1994
	All Engines
	R134a
	40.00
	PAG 100
	5.00

	1989-1992
	All Engines
	R12
	40.00
	Mineral
	7.00

	
	XJ8
	

	2001-2002
	All Engines
	R134a
	24.00
	PAG 150
	5.5-6.5

	1998-2000
	All Engines
	R134a
	26.50
	PAG 150
	6.75

	
	XJR
	

	2001-2002
	All Engines
	R134a
	24.00
	PAG 150
	5.5-6.5

	1998-2000
	All Engines
	R134a
	40.00
	PAG 150
	5.00

	
	XJRS
	

	1993
	All Engines
	R134a
	40.00
	PAG 150
	5.00

	
	XJS
	

	1993-1996
	All Engines
	R134a
	40.00
	PAG 150
	6.50

	1990-1992
	All Engines
	R12
	40.00
	Mineral
	7.00

	1989
	All Engines - w/ Delco Compressor ...
	R12
	40.00
	Mineral
	7.00

	
	All Engines - w/ Sanden Compressor...
	R12
	56.00
	Mineral
	6.50

	
	XK8
	

	2001-2002
	All Engines
	R134a
	21.0 - 23.0
	PAG 150
	5.5-6.5

	1999-2000
	All Engines
	R134a
	26.50
	PAG 150
	6.50

	1997-1998
	All Engines
	R134a
	32.00
	PAG 150
	6.50

	
	XKR
	

	2001-2002
	All Engines
	R134a
	21.0 - 23.0
	PAG 150
	5.5-6.5


Jeep
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	CHEROKEE
	

	2000-2001
	All Engines
	R134a
	20.00
	PAG 100
	6.00

	1997-1999
	All Engines
	R134a
	20.00
	PAG 100
	8.00

	1995-1996
	All Engines
	R134a
	32.00
	PAG 100
	7.75

	1990-1994
	All Engines
	R12
	38.00
	Mineral
	4.50

	1989
	All Engines
	R12
	32.00
	Mineral
	6.00

	1986-1988
	All Engines
	R12
	36.00
	Mineral
	8.00

	
	COMANCHE
	

	1990-1992
	All Engines
	R12
	38.00
	Mineral
	4.50

	1989
	All Engines
	R12
	32.00
	Mineral
	6.00

	
	GRAND CHEROKEE
	

	2002-2004
	All Engines
	R134a
	27.00
	PAG 46
	4.50

	2000-2001
	All Engines
	R134a
	24.00
	PAG 100
	5.75

	1998-1999
	All Engines
	R134a
	26.00
	PAG 100
	7.50

	1993-1997
	All Engines
	R134a
	28.00
	PAG 100
	7.75

	
	GRAND WAGONEER
	

	1993
	All Engines
	R12
	28.00
	Mineral
	7.75

	1989-1991
	All Engines
	R12
	32.00
	Mineral
	4.50

	
	LIBERTY
	

	2002-2003
	All Engines
	R134a
	26.00
	PAG 46
	8.00

	
	WAGONEER
	

	1990
	All Engines
	R12
	38.00
	Mineral
	4.50

	1989
	All Engines
	R12
	32.00
	Mineral
	6.00

	
	WRANGLER
	

	2001-2004
	All Engines
	R134a
	20.00
	PAG 46
	6.00

	2000
	All Engines
	R134a
	20.00
	PAG 100
	6.00

	1997-1999
	All Engines
	R134a
	20.00
	PAG 100
	8.00

	1995
	All Engines
	R134a
	32.00
	PAG 100
	4.50

	1990-1994
	All Engines
	R12
	32.00
	Mineral
	4.50

	1989
	All Engines
	R12
	32.00
	Mineral
	6.00


Kia
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	AMANTI
	

	2004
	All Engines
	R134a
	23.00
	PAG 46
	5.50

	
	OPTIMA
	

	2001-2004
	All Engines
	R134a
	22.0 - 24.0
	PAG 46
	5.00

	
	RIO
	

	2001-2004
	All Engines
	R134a
	22.0 - 24.0
	PAG 46
	5.00

	
	SEDONA
	

	2002-2004
	All Engines
	R134a
	34.4 - 36.1
	PAG 46
	7.00

	
	SEPHIA
	

	1998-2001
	All Engines
	R134a
	26.50
	PAG 46
	6.75

	1994-1997
	All Engines
	R134a
	24.00
	--
	--

	
	SORENTO
	

	2003-2004
	All Engines
	R134a
	22.00
	PAG 46
	5.00

	
	SPECTRA
	

	2002-2003
	All Engines
	R134a
	22.0 - 24.0
	PAG 46
	6.00

	2000-2001
	All Engines
	R134a
	22.9 - 26.5
	PAG 46
	6.00

	
	SPORTAGE
	

	1995-2001
	All Engines
	R134a
	25.00
	PAG 100
	--


Land Rover
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	DEFENDER SERIES
	

	1997
	All Engines
	R134a
	32.00
	PAG 46
	6.50

	1995
	All Engines
	R134a
	32.00
	PAG 46
	5.00

	1993-1994
	All Engines
	R12
	38.00
	Mineral
	5.00

	
	DISCOVERY
	

	2000-2001
	All Engines w/ Front & Rear AC ...
	R134a
	32.00
	PAG 46
	6.50

	
	All Engines w/ Front AC Only ...
	R134a
	25.00
	PAG 46
	6.50

	1994-1999
	All Engines w/ Front & Rear AC ...
	R134a
	41.00
	PAG 46
	6.50

	
	All Engines w/ Front AC Only ...
	R134a
	32.00
	PAG 46
	6.50

	
	RANGE ROVER
	

	2000-2001
	All Engines
	R134a
	49.00
	PAG 46
	6.00

	1999
	All Engines
	R134a
	41.00
	PAG 46
	6.00

	1998
	New Body Style ...
	R134a
	44.00
	PAG 46
	5.00

	1996-1997
	New Body Style ...
	R134a
	44.00
	PAG 46
	5.00

	
	Old Body Style ...
	R134a
	32.00
	PAG 46
	5.00

	1995
	Old Body Style ...
	R134a
	32.00
	PAG 46
	5.00


Lexus
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	ES330
	

	2004
	All Engines
	R134a
	19.00
	PAG 46
	4.00

	
	ES250
	

	1990-1991
	All Engines
	R12
	27.00
	Mineral
	7.00

	
	ES300
	

	2002-2003
	All Engines
	R134a
	19.00
	PAG 46
	--

	1998-2001
	All Engines
	R134a
	26.5 - 30.0
	PAG 46
	7.00

	1994-1997
	All Engines
	R134a
	30.00
	PAG 46
	7.75

	1993
	All Engines
	R12
	33.50
	Mineral
	7.00

	1992
	All Engines
	R12
	35.00
	Mineral
	7.00

	
	GS300
	

	2002-2003
	All Engines
	R134a
	19.00
	PAG 46
	--

	1998-2001
	All Engines
	R134a
	21.0 - 23.0
	PAG 46
	7.00

	1993-1997
	All Engines
	R134a
	30.00
	PAG 46
	7.00

	
	GS400
	

	1998-2000
	All Engines
	R134a
	21.0 - 23.0
	PAG 46
	7.00

	
	GS430
	

	2001-2004
	All Engines
	R134a
	21.0 - 23.0
	PAG 46
	7.00

	
	GX470
	

	2003-2004
	All Engines w/ Front AC Only ...
	R134a
	22.4 - 24.0
	PAG 46
	4.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	28.1 - 28.9
	PAG 46
	4.50

	
	IS300
	

	2001-2002
	All Engines
	R134a
	23.00
	PAG 46
	--

	
	LS400
	

	1998-2000
	All Engines
	R134a
	22.9 - 26.5
	PAG 46
	7.75

	1995-1997
	All Engines
	R134a
	30.00
	PAG 46
	7.75

	1993
	All Engines
	R134a
	33.50
	PAG 46
	5.25

	1990-1992
	All Engines
	R12
	37.00
	Mineral
	7.00

	
	LS430
	

	2004
	All Engines w/ Front & Rear AC ...
	R134a
	30.0 - 33.5
	PAG 46
	--

	2003
	All Engines w/ Front & Rear AC ...
	R134a
	30.0 - 33.5
	PAG 46
	--

	
	All Engines w/ Front AC Only ...
	R134a
	22.0 - 24.0
	PAG 46
	--

	
	All Engines w/ Front AC Only ...
	R134a
	22.0 - 24.0
	PAG 46
	--

	2001-2002
	All Engines w/ Front AC Only ...
	R134a
	22.0 - 24.0
	PAG 46
	--

	
	All Engines w/ Front & Rear AC ...
	R134a
	30.0 - 33.5
	PAG 46
	--

	
	LX450
	

	1996-1997
	All Engines
	R134a
	30.00
	PAG 46
	7.00

	
	LX470
	

	1998-2004
	All Engines
	R134a
	35.3 - 38.8
	PAG 46
	7.75

	
	RX300
	

	1999-2001
	All Engines
	R134a
	22.0 - 24.0
	PAG 46
	7.75

	
	RX330
	

	2004
	All Engines
	R134a
	22.0 - 24.0
	PAG 46
	4.00

	
	SC300
	

	1997-2000
	All Engines
	R134a
	32.0 - 34.0
	PAG 46
	7.00

	1996
	All Engines
	R134a
	31.8 - 35.3
	PAG 46
	7.00

	1994-1995
	All Engines
	R134a
	30.00
	PAG 46
	7.75

	1993
	All Engines
	R12
	33.50
	Mineral
	7.00

	1992
	All Engines
	R12
	35.00
	Mineral
	7.00

	
	SC400
	

	1996-2000
	All Engines
	R134a
	33.50
	PAG 46
	7.00

	1994-1995
	All Engines
	R134a
	30.00
	PAG 46
	7.75

	1993
	All Engines
	R12
	33.50
	Mineral
	7.00

	1992
	All Engines
	R12
	35.00
	Mineral
	7.00

	
	SC430
	

	2002-2004
	All Engines
	R134a
	21.0 - 23.0
	PAG 46
	4.00


Lincoln
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	AVIATOR
	

	2003-2004
	All Engines w/ Front & Rear AC ...
	R134a
	56.00
	PAG 46
	13.00

	
	BLACKWOOD
	

	2002-2004
	All Engines
	R134a
	37.00
	PAG 46
	9.00

	
	CONTINENTAL
	

	1998-2002
	All Engines
	R134a
	38.00
	PAG 46
	7.00

	1995-1997
	All Engines
	R134a
	34.00
	PAG 46
	7.00

	1994
	All Engines
	R134a
	40.00
	PAG 46
	7.00

	1993
	All Engines w/ Front & Rear AC ...
	R12
	40.00
	Mineral
	10.00

	
	All Engines w/ Front AC Only ...
	R12
	40.00
	Mineral
	7.00

	1992
	All Engines
	R12
	40.00
	Mineral
	7.00

	1989-1991
	All Engines
	R12
	40.00
	Mineral
	8.00

	
	LS
	

	2000-2004
	All Engines
	R134a
	28.00
	PAG 46
	7.00

	
	MARK SERIES
	

	1995-1998
	Mark VII, Mark VIII ...
	R134a
	34.00
	PAG 46
	7.00

	1994
	Mark VII, Mark VIII ...
	R134a
	40.00
	PAG 46
	7.00

	1993
	Mark VII, Mark VIII w/ Front & Rear AC...
	R134a
	34.00
	PAG 46
	10.00

	
	Mark VII, Mark VIII w/ Front AC Only ...
	R134a
	34.00
	PAG 46
	7.00

	1989-1992
	Mark VII, Mark VIII ...
	R12
	40.00
	Mineral
	8.00

	1988
	All Engines
	R12
	40.00
	Mineral
	10.00

	
	NAVIGATOR
	

	2003-2004
	All Engines w/ Front & Rear AC ...
	R134a
	54.00
	PAG 46
	14.00

	
	All Engines w/ Front AC Only ...
	R134a
	32.00
	PAG 46
	9.00

	1998-2002
	All Engines w/ Front & Rear AC ...
	R134a
	62.00
	PAG 46
	14.00

	
	All Engines w/ Front AC Only ...
	R134a
	37.00
	PAG 46
	9.00

	
	TOWN CAR
	

	2003-2004
	All Engines
	R134a
	32.00
	PAG 46
	8.00

	1998-2002
	All Engines
	R134a
	38.00
	PAG 46
	7.50

	1997
	All Engines
	R134a
	34.00
	PAG 46
	7.50

	1995-1996
	All Engines
	R134a
	34.00
	PAG 46
	7.00

	1994
	All Engines w/ Front & Rear AC ...
	R134a
	40.00
	PAG 46
	8.00

	
	All Engines w/ Front AC Only ...
	R134a
	40.00
	PAG 46
	7.00

	1993
	All Engines w/ Front & Rear AC ...
	R12
	36.00
	Mineral
	10.00

	
	All Engines w/ Front AC Only ...
	R12
	36.00
	Mineral
	7.00

	1991-1992
	All Engines
	R12
	40.00
	Mineral
	7.00

	1989-1990
	All Engines
	R12
	48.00
	Mineral
	8.00

	1988
	All Engines
	R12
	52.00
	Mineral
	10.00

	1986-1987
	All Engines
	R12
	48.00
	Mineral
	10.00

	1985
	All Engines
	R12
	52.00
	Mineral
	10.00


Mazda
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	MAZDA 3
	

	2004
	All Engines
	R134a
	18.00
	PAG 100
	5.00

	
	323
	

	1993-1994
	All Engines
	R134a
	28.00
	PAG 46
	7.50

	1989-1992
	All Engines
	R12
	27.00
	Mineral
	6.75

	1988
	All Engines
	R12
	27.00
	Mineral
	7.75

	1986-1987
	All Engines
	R12
	30.00
	Mineral
	7.75

	
	MAZDA 6
	

	2003-2004
	All Engines
	R134a
	16.00
	PAG 100
	5.00

	
	626
	

	1998-2002
	All Engines
	R134a
	26.50
	PAG 100
	8.00

	1994-1997
	All Engines
	R134a
	25.00
	PAG 100
	8.00

	1989-1993
	All Engines
	R12
	26.00
	Mineral
	6.00

	1988
	All Engines
	R12
	37.00
	Mineral
	7.75

	1986-1987
	All Engines
	R12
	32.00
	Mineral
	7.75

	1985
	All Engines
	R12
	--
	Mineral
	7.75

	
	929
	

	1994-1995
	All Engines
	R134a
	28.00
	PAG 100
	7.50

	1993
	All Engines
	R12
	28.00
	Mineral
	6.50

	1989-1991
	All Engines
	R12
	40.00
	Mineral
	8.00

	
	MIATA
	

	2001-2004
	All Engines
	R134a
	14.00
	--
	--

	1999-2000
	All Engines
	R134a
	20.00
	PAG 100
	8.75

	1994-1997
	All Engines
	R134a
	21.00
	PAG 100
	8.75

	1993
	All Engines
	R12
	28.00
	Mineral
	6.50

	1990-1992
	All Engines
	R12
	29.00
	Mineral
	6.50

	
	MILLENIA
	

	1995-2002
	All Engines
	R134a
	26.50
	PAG 100
	6.00

	
	MPV
	

	2003-2004
	All Engines w/ Front AC Only ...
	R134a
	21.00
	PAG 46
	8.50

	
	All Engines w/ Front & Rear AC ...
	R134a
	28.00
	PAG 46
	8.50

	2000-2002
	All Engines w/ Front & Rear AC ...
	R134a
	30.00
	PAG 46
	8.50

	
	All Engines w/ Front AC Only ...
	R134a
	23.00
	PAG 46
	8.50

	1997-1998
	All Engines w/ Front & Rear AC ...
	R134a
	35.00
	PAG 46
	6.75

	
	All Engines w/ Front AC Only ...
	R134a
	32.00
	PAG 46
	6.75

	1995-1996
	All Engines w/ Front & Rear AC ...
	R134a
	35.00
	PAG 46
	8.75

	
	All Engines w/ Front AC Only ...
	R134a
	32.00
	PAG 46
	7.25

	1994
	All Engines w/ Front & Rear AC ...
	R134a
	44.00
	PAG 46
	8.75

	
	All Engines w/ Front AC Only ...
	R134a
	37.00
	PAG 46
	8.75

	1991-1993
	All Engines w/ Front & Rear AC ...
	R12
	49.50
	Mineral
	6.50

	
	All Engines w/ Front AC Only ...
	R12
	43.00
	Mineral
	6.50

	1990
	All Engines w/ Front AC Only ...
	R12
	42.00
	Mineral
	6.50

	
	All Engines w/ Front & Rear AC ...
	R12
	49.00
	Mineral
	6.50

	
	MX-3
	

	1994-1995
	All Engines
	R134a
	26.50
	PAG 100
	6.25

	1993
	All Engines
	R12
	28.00
	Mineral
	4.50

	
	MX-6
	

	1997
	All Engines
	R134a
	24.00
	PAG 100
	6.00

	1994-1996
	All Engines
	R134a
	24.00
	PAG 100
	5.25

	1989-1993
	All Engines
	R12
	26.00
	Mineral
	6.00

	1988
	All Engines
	R12
	37.00
	Mineral
	7.75

	
	NAVAJO
	

	1994
	All Engines w/ Front AC Only ...
	R134a
	28.00
	PAG 100
	7.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	28.00
	PAG 100
	10.00

	1991-1993
	All Engines
	R12
	28.00
	Mineral
	7.00

	
	PICKUP
	

	2001-2004
	B2500 ...
	R134a
	30.00
	PAG 46
	9.00

	1998-2000
	B2500 ...
	R134a
	30.00
	PAG 46
	9.00

	
	B3000 ...
	R134a
	30.00
	PAG 46
	9.00

	
	B4000 ...
	R134a
	30.00
	PAG 46
	9.00

	1997
	B2300 ...
	R134a
	30.00
	PAG 46
	9.00

	
	B3000 ...
	R134a
	30.00
	PAG 46
	9.00

	
	B4000 ...
	R134a
	30.00
	PAG 46
	9.00

	1994-1996
	B2300 ...
	R134a
	22.00
	PAG 46
	7.00

	
	B3000 ...
	R134a
	22.00
	PAG 46
	7.00

	
	B4000 ...
	R134a
	22.00
	PAG 46
	7.00

	1990-1991
	B2200 ...
	R12
	26.50
	Mineral
	8.00

	1987-1989
	B2200 ...
	R12
	30.00
	Mineral
	8.00

	1986
	B2000 ...
	R12
	30.00
	Mineral
	8.00

	
	PROTÃ‰GÃ‰
	

	2003
	All Engines
	R134a
	19.00
	PAG 100
	--

	1999-2002
	All Engines
	R134a
	21.00
	PAG 100
	8.00

	1995-1998
	All Engines
	R134a
	21.00
	PAG 46
	7.00

	1993-1994
	All Engines
	R134a
	28.00
	PAG 46
	7.50

	1990-1992
	All Engines
	R12
	24.0 - 27.0
	Mineral
	4.00

	
	RX-7
	

	1994-1995
	All Engines
	R134a
	19.00
	PAG 100
	5.75

	1993
	All Engines
	R12
	22.00
	Mineral
	5.00

	1989-1991
	All Engines - w/ Denso Compressor ...
	R12
	29.00
	Mineral
	6.00

	
	All Engines - w/ Sanden Compressor...
	R12
	29.00
	Mineral
	6.00

	
	RX-8
	

	2004
	All Engines
	R134a
	16.00
	PAG 46
	--

	
	TRIBUTE
	

	2001-2004
	All Engines
	R134a
	32.00
	PAG 46
	7.00


Mercedes-Benz
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	190 SERIES
	

	1990-1993
	190E ...
	R12
	35.00
	Mineral
	7.75

	1989
	190D ...
	R12
	35.00
	Mineral
	7.50

	
	190E ...
	R12
	35.00
	Mineral
	7.75

	
	300 SERIES
	

	1993
	300CE ...
	R134a
	33.50
	PAG 46
	8.50

	
	300D ...
	R134a
	33.50
	PAG 46
	8.50

	
	300E ...
	R134a
	33.50
	PAG 46
	8.50

	
	300SD w/ Front & Rear AC ...
	R134a
	41.50
	PAG 46
	8.50

	
	300SD w/ Front AC Only ...
	R134a
	39.00
	PAG 46
	8.50

	1992
	300CE ...
	R12
	35.00
	Mineral
	7.75

	
	300D ...
	R12
	46.00
	Mineral
	7.75

	
	300E ...
	R12
	35.00
	Mineral
	7.75

	
	300SD w/ Front & Rear AC ...
	R134a
	41.50
	PAG 46
	8.50

	
	300SD w/ Front AC Only ...
	R134a
	39.00
	PAG 46
	8.50

	1991
	300CE ...
	R12
	35.00
	Mineral
	7.75

	
	300D ...
	R12
	46.00
	Mineral
	7.75

	
	300E ...
	R12
	35.00
	Mineral
	7.75

	
	300SE ...
	R12
	46.00
	Mineral
	7.75

	
	300SEL ...
	R12
	46.00
	Mineral
	7.75

	1989-1990
	300CE ...
	R12
	35.00
	Mineral
	7.75

	
	300E ...
	R12
	35.00
	Mineral
	7.75

	
	300SE ...
	R12
	46.00
	Mineral
	7.75

	
	300SEL ...
	R12
	46.00
	Mineral
	7.75

	
	350 SERIES
	

	1991
	All Engines
	R12
	46.00
	Mineral
	7.75

	1990
	350SDL ...
	R12
	46.00
	Mineral
	7.75

	
	400 SERIES
	

	1993
	400E ...
	R134a
	33.50
	PAG 46
	8.50

	
	400SEL w/ Front & Rear AC ...
	R134a
	41.50
	PAG 46
	8.50

	
	400SEL w/ Front AC Only ...
	R134a
	39.00
	PAG 46
	8.50

	1992
	400E w/ Front & Rear AC ...
	R12
	44.00
	Mineral
	7.75

	
	400E w/ Front AC Only ...
	R12
	39.00
	Mineral
	7.75

	
	400SE w/ Front & Rear AC ...
	R134a
	41.50
	PAG 46
	8.50

	
	400SE w/ Front AC Only ...
	R134a
	39.00
	PAG 46
	8.50

	
	420 SERIES
	

	1989-1991
	420SEL ...
	R12
	46.00
	Mineral
	7.75

	
	500 SERIES
	

	1993
	500E ...
	R134a
	33.50
	PAG 46
	8.50

	
	500SEC w/ Front & Rear AC ...
	R134a
	41.50
	PAG 46
	8.50

	
	500SEC w/ Front AC Only ...
	R134a
	39.00
	PAG 46
	8.50

	
	500SEL w/ Front & Rear AC ...
	R134a
	41.50
	PAG 46
	8.50

	
	500SEL w/ Front AC Only ...
	R134a
	39.00
	PAG 46
	8.50

	1992
	500E w/ Front & Rear AC ...
	R12
	44.00
	Mineral
	7.75

	
	500E w/ Front AC Only ...
	R12
	39.00
	Mineral
	7.75

	1992
	500SEL w/ Front & Rear AC ...
	R134a
	41.50
	PAG 46
	8.50

	
	500SEL w/ Front AC Only ...
	R134a
	39.00
	PAG 46
	8.50

	
	560 SERIES
	

	1989-1991
	560SEC ...
	R12
	46.00
	Mineral
	7.75

	
	560SEL ...
	R12
	46.00
	Mineral
	7.75

	
	600 SERIES
	

	1993
	600SEC w/ Front & Rear AC ...
	R134a
	41.50
	PAG 46
	8.50

	
	600SEC w/ Front AC Only ...
	R134a
	39.00
	PAG 46
	8.50

	
	600SEL w/ Front & Rear AC ...
	R134a
	41.50
	PAG 46
	8.50

	
	600SEL w/ Front AC Only ...
	R134a
	39.00
	PAG 46
	8.50

	
	600SL w/ Front AC Only ...
	R134a
	33.50
	PAG 46
	5.00

	1992
	600SEL w/ Front & Rear AC ...
	R134a
	41.50
	PAG 46
	8.50

	
	600SEL w/ Front AC Only ...
	R134a
	39.00
	PAG 46
	8.50

	
	C CLASS
	

	2001-2003
	All Engines
	R134a
	26.50
	PAG 46
	4.00

	1995-2000
	All Engines
	R134a
	29.00
	PAG 46
	8.00

	1994
	All Engines
	R134a
	28.80
	PAG 46
	8.20

	
	CL CLASS
	

	2001-2003
	CL500 w/ Front & Rear AC ...
	R134a
	33.50
	PAG 46
	4.00

	1998-2000
	CL500 w/ Front & Rear AC ...
	R134a
	41.50
	PAG 46
	7.00

	
	CL500 w/ Front AC Only ...
	R134a
	39.00
	PAG 46
	6.00

	
	CL600 w/ Front & Rear AC ...
	R134a
	41.50
	PAG 46
	7.00

	
	CL600 w/ Front AC Only ...
	R134a
	39.00
	PAG 46
	6.00

	
	CLK CLASS
	

	2001-2003
	CLK320 ...
	R134a
	30.00
	PAG 46
	4.00

	1999-2000
	CLK320 ...
	R134a
	30.00
	PAG 46
	7.00

	
	CLK430 ...
	R134a
	30.00
	PAG 46
	7.00

	1998
	CLK320 ...
	R134a
	30.00
	PAG 46
	7.00

	
	E CLASS
	

	2001-2003
	E320 ...
	R134a
	35.00
	PAG 46
	7.00

	1998-2000
	E320 ...
	R134a
	33.50
	PAG 46
	8.50

	
	E430 ...
	R134a
	33.50
	PAG 46
	8.50

	1997
	E320 ...
	R134a
	33.50
	PAG 46
	8.50

	
	E420 ...
	R134a
	33.50
	PAG 46
	8.50

	1996
	E320 ...
	R134a
	33.50
	PAG 46
	8.50

	1995
	E320 ...
	R134a
	33.50
	PAG 46
	8.00

	
	E420 ...
	R134a
	33.50
	PAG 46
	8.00

	1994
	E320 ...
	R134a
	33.50
	PAG 46
	8.00

	
	E420 ...
	R134a
	33.50
	PAG 46
	8.00

	
	E500 ...
	R134a
	33.50
	PAG 46
	8.00

	
	ML CLASS
	

	2001-2003
	ML320 ...
	R134a
	26.50
	PAG 46
	6.00

	1999-2000
	ML320 ...
	R134a
	27.00
	PAG 46
	8.00

	
	ML430 ...
	R134a
	27.00
	PAG 46
	8.00

	1998
	ML320 ...
	R134a
	27.00
	PAG 46
	8.00

	
	S CLASS
	

	2001-2003
	S500 w/ Front & Rear AC ...
	R134a
	37.00
	PAG 46
	6.00

	
	S500 w/ Front AC Only ...
	R134a
	33.50
	PAG 46
	6.00

	2000
	S500 w/ Front & Rear AC ...
	R134a
	41.50
	PAG 46
	8.50

	
	S500 w/ Front AC Only ...
	R134a
	39.00
	PAG 46
	8.00

	1996-1999
	All Engines w/ Front & Rear AC ...
	R134a
	41.50
	PAG 46
	8.50

	
	All Engines w/ Front AC Only ...
	R134a
	39.00
	PAG 46
	8.00

	1994-1995
	S320 w/ Front & Rear AC ...
	R134a
	41.50
	PAG 46
	8.50

	
	S320 w/ Front AC Only ...
	R134a
	39.00
	PAG 46
	8.00

	
	S420 w/ Front & Rear AC ...
	R134a
	41.50
	PAG 46
	8.50

	
	S420 w/ Front AC Only ...
	R134a
	39.00
	PAG 46
	8.00

	
	S500 w/ Front & Rear AC ...
	R134a
	41.50
	PAG 46
	8.50

	
	S500 w/ Front AC Only ...
	R134a
	39.00
	PAG 46
	8.00

	
	S600 w/ Front & Rear AC ...
	R134a
	41.50
	PAG 46
	8.50

	
	S600 w/ Front AC Only ...
	R134a
	39.00
	PAG 46
	8.00

	
	SL CLASS
	

	2001-2003
	SL500 ...
	R134a
	29.00
	PAG 46
	5.00

	1998-2000
	SL500 ...
	R134a
	32.00
	PAG 46
	8.50

	
	SL600 ...
	R134a
	32.00
	PAG 46
	8.50

	1994-1997
	SL320 ...
	R134a
	32.00
	PAG 46
	8.50

	
	SL500 ...
	R134a
	32.00
	PAG 46
	8.50

	
	SL600 ...
	R134a
	32.00
	PAG 46
	8.50

	
	SLK CLASS
	

	2001-2003
	SLK230 ...
	R134a
	30.00
	PAG 46
	4.00

	1998-2000
	SLK230 ...
	R134a
	29.00
	PAG 46
	8.50


Mercury
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	CAPRI
	

	1994
	All Engines
	R134a
	25.00
	PAG 46
	10.00

	1991-1993
	All Engines
	R12
	25.00
	Mineral
	10.00

	
	COUGAR
	

	2000-2003
	All Engines
	R134a
	26.00
	PAG 46
	7.00

	1999
	All Engines
	R134a
	26.00
	PAG 46
	6.75

	1995-1997
	All Engines
	R134a
	36.00
	PAG 46
	7.00

	1994
	All Engines
	R134a
	36.00
	PAG 46
	6.75

	1989-1993
	All Engines
	R12
	40.00
	Mineral
	7.00

	1987-1988
	All Engines
	R12
	40.00
	Mineral
	8.00

	1986
	All Engines To 1/86 ...
	R12
	40.00
	Mineral
	10.00

	
	All Engines From 1/86 ...
	R12
	40.00
	Mineral
	8.00

	
	GRAND MARQUIS
	

	1997-2004
	All Engines
	R134a
	38.00
	PAG 46
	7.50

	1995-1996
	All Engines
	R134a
	38.00
	PAG 46
	7.00

	1994
	All Engines
	R134a
	34.00
	PAG 46
	7.00

	1993
	All Engines
	R12
	38.00
	Mineral
	7.00

	1992
	All Engines
	R12
	36.00
	Mineral
	7.00

	1989-1991
	All Engines
	R12
	48.00
	Mineral
	8.00

	1988
	All Engines
	R12
	52.00
	Mineral
	13.00

	1987
	All Engines To 9/86 ...
	R12
	52.00
	Mineral
	10.00

	
	All Engines From 9/86 ...
	R12
	52.00
	Mineral
	13.00

	1985-1986
	All Engines
	R12
	52.00
	Mineral
	10.00

	
	MOUNTAINEER
	

	2002-2004
	All Engines w/ Front AC Only ...
	R134a
	34.00
	PAG 46
	9.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	56.00
	PAG 46
	13.00

	1998-2001
	All Engines
	R134a
	30.00
	PAG 46
	9.00

	1997
	All Engines
	R134a
	22.00
	PAG 46
	9.00

	
	MYSTIQUE
	

	1995-2001
	All Engines
	R134a
	26.00
	PAG 46
	7.00

	
	SABLE
	

	1996-2003
	All Engines
	R134a
	34.00
	PAG 46
	6.50

	1994-1995
	All Engines
	R134a
	32.00
	PAG 46
	7.00

	1993
	All Engines
	R12
	32.00
	Mineral
	7.00

	1991-1992
	All Engines
	R12
	40.00
	Mineral
	7.00

	1989-1990
	All Engines
	R12
	40.00
	Mineral
	8.00

	1986-1988
	All Engines
	R12
	44.00
	Mineral
	10.00

	
	TOPAZ
	

	1994
	All Engines
	R134a
	32.00
	PAG 46
	7.00

	1993
	All Engines
	R12
	37.00
	Mineral
	8.00

	1989-1992
	All Engines
	R12
	36.00
	Mineral
	7.00

	1987-1988
	All Engines
	R12
	36.00
	Mineral
	8.00

	
	TRACER
	

	1999
	All Engines
	R134a
	27.00
	PAG 46
	7.00

	1998
	All Engines
	R134a
	28.00
	PAG 46
	7.00

	1997
	All Engines
	R134a
	28.00
	PAG 46
	9.00

	1994-1996
	All Engines
	R134a
	28.00
	PAG 46
	8.00

	1993
	All Engines
	R12
	34.00
	Mineral
	7.75

	1991-1992
	All Engines
	R12
	34.00
	Mineral
	8.00

	1989-1990
	All Engines
	R12
	36.00
	Mineral
	8.00

	1988
	All Engines
	R12
	31.00
	Mineral
	10.00

	
	VILLAGER
	

	2000-2002
	All Engines w/ Front & Rear AC ...
	R134a
	52.00
	PAG 100
	11.00

	
	All Engines w/ Front AC Only ...
	R134a
	32.00
	PAG 100
	7.00

	1999
	All Engines w/ Front & Rear AC ...
	R134a
	52.00
	PAG 100
	11.00

	
	All Engines w/ Front AC Only ...
	R134a
	32.00
	PAG 100
	10.00

	1998
	All Engines w/ Front & Rear AC ...
	R134a
	52.00
	PAG 100
	10.00

	
	All Engines w/ Front AC Only ...
	R134a
	32.00
	PAG 100
	10.00

	1994-1997
	All Engines w/ Front & Rear AC ...
	R134a
	52.00
	PAG 100
	10.00

	
	All Engines w/ Front AC Only ...
	R134a
	32.00
	PAG 100
	7.00

	1993
	All Engines w/ Front & Rear AC ...
	R12
	56.00
	Mineral
	10.00

	
	All Engines w/ Front AC Only ...
	R12
	36.00
	Mineral
	7.00


Merkur
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	SCORPIO
	

	1989
	All Engines
	R12
	40.00
	Mineral
	8.00

	
	XR4TI
	

	1989
	All Engines
	R12
	40.00
	Mineral
	8.00

	MINI
	
	

	
	
	Refrigerant
	
	Oil
	

	Year
	Model
	Type
	Ounces
	Type
	Ounces

	
	COOPER
	

	2002-2004
	All Engines
	R134a
	14.00
	--
	--


Mitsubishi
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	3000GT
	

	1994-1999
	All Engines From 10/92 ...
	R134a
	28.00
	PAG 46
	6.50

	1993
	All Engines From 10/92 ...
	R12
	29.00
	Mineral
	7.00

	1992
	All Engines From 10/92 ...
	R12
	29.00
	Mineral
	7.00

	
	All Engines To 9/92 ...
	R12
	34.00
	Mineral
	7.00

	1991
	All Engines
	R12
	34.00
	Mineral
	8.50

	
	DIAMANTE
	

	2000-2004
	All Engines
	R134a
	23.00
	PAG 46
	5.50

	1997-1999
	All Engines
	R134a
	24.00
	PAG 46
	6.50

	1996
	All Engines
	R134a
	28.00
	PAG 46
	6.50

	1994-1995
	Sedan ...
	R134a
	28.00
	PAG 46
	6.50

	
	Wagon ...
	R134a
	28.00
	PAG 46
	8.50

	1993
	Sedan ...
	R12
	38.00
	Mineral
	6.00

	
	Wagon ...
	R12
	28.00
	Mineral
	8.50

	1992
	All Engines
	R12
	38.00
	Mineral
	6.00

	
	ECLIPSE
	

	2000-2004
	All Engines
	R134a
	15.00
	PAG 46
	5.50

	1995-1999
	All Engines
	R134a
	26.00
	PAG 46
	6.50

	1990-1994
	All Engines
	R12
	33.00
	Mineral
	6.50

	
	ENDEAVOR
	

	2004
	All Engines
	R134a
	21.00
	PAG 46
	4.75

	
	EXPO
	

	1994-1995
	All Engines
	R134a
	27.00
	PAG 46
	6.50

	1993
	4 Cyl. 1.8 Eng. ..
	R12
	30.00
	Mineral
	7.00

	
	4 Cyl. 2.4 Eng. ..
	R12
	30.00
	Mineral
	6.50

	1992
	All Engines
	R12
	30.00
	Mineral
	6.50

	
	GALANT
	

	2000-2004
	All Engines
	R134a
	23.00
	PAG 46
	5.50

	1997-1999
	All Engines
	R134a
	24.00
	PAG 46
	7.00

	1994-1996
	All Engines
	R134a
	24.00
	PAG 46
	8.75

	1989-1993
	All Engines
	R12
	33.00
	Mineral
	8.75

	
	LANCER
	

	2003-2004
	All Engines
	R134a
	18.00
	PAG 46
	4.75

	2002
	All Engines
	R134a
	18.00
	PAG 46
	6.50

	
	MIGHTY MAX
	

	1994-1996
	All Engines
	R134a
	28.00
	PAG 46
	7.75

	1990-1993
	All Engines
	R12
	30.00
	Mineral
	8.50

	
	MIRAGE
	

	2002
	All Engines
	R134a
	23.00
	PAG 46
	4.50

	1997-2001
	All Engines
	R134a
	21.00
	PAG 46
	4.50

	1994-1996
	All Engines
	R134a
	30.00
	PAG 46
	8.00

	1993
	All Engines
	R12
	30.00
	Mineral
	8.00

	1989-1992
	All Engines
	R12
	36.00
	Mineral
	8.06

	
	MONTERO
	

	2002-2003
	All Engines w/ Front & Rear AC ...
	R134a
	27.00
	PAG 46
	5.00

	
	All Engines w/ Front AC Only ...
	R134a
	21.00
	PAG 46
	4.00

	2001
	All Engines w/ Front & Rear AC ...
	R134a
	33.00
	PAG 46
	5.00

	
	All Engines w/ Front AC Only ...
	R134a
	23.00
	PAG 46
	4.00

	1994-2000
	All Engines
	R134a
	23.00
	PAG 46
	5.75

	1992-1993
	All Engines
	R12
	28.00
	Mineral
	5.75

	1989-1991
	All Engines
	R12
	32.00
	Mineral
	6.75

	
	MONTERO SPORT
	

	1997-2004
	All Engines
	R134a
	24.00
	PAG 46
	8.75

	
	OUTLANDER
	

	2003-2004
	All Engines
	R134a
	19.00
	PAG 46
	4.75

	
	PRECIS
	

	1990-1992
	All Engines
	R12
	30.00
	Mineral
	8.00

	1989
	All Engines
	R12
	35.00
	Mineral
	5.50

	
	SIGMA
	

	1990
	All Engines
	R12
	32.00
	Mineral
	8.00

	1989
	All Engines
	R12
	32.00
	Mineral
	8.75

	
	STARION
	

	1989
	All Engines
	R12
	29.00
	Mineral
	8.75

	
	VAN
	

	1987-1990
	All Engines w/ Front & Rear AC ...
	R12
	52.00
	Mineral
	5.00

	
	All Engines w/ Front AC Only ...
	R12
	32.00
	Mineral
	4.00


Nissan
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	200 SERIES
	

	1995-1998
	200SX ...
	R134a
	25.00
	PAG 46
	6.75

	
	240 SERIES
	

	1995-1998
	240SX ...
	R134a
	25.00
	PAG 46
	8.50

	1991-1993
	240SX ...
	R12
	32.00
	Mineral
	8.00

	1989-1990
	240SX ...
	R12
	35.00
	Mineral
	6.75

	
	300 SERIES
	

	1994-1996
	300ZX ...
	R134a
	23.00
	PAG 46
	6.75

	1991-1993
	300ZX ...
	R12
	30.00
	Mineral
	6.75

	1990
	300ZX ...
	R12
	34.00
	Mineral
	6.75

	1989
	300ZX ...
	R12
	38.50
	Mineral
	6.75

	1985-1986
	300ZX ...
	R12
	38.00
	Mineral
	5.00

	
	350Z
	

	2003-2004
	350Z ...
	R134a
	19.00
	PAG 100
	6.00

	
	720 PICKUP
	

	1986
	All Engines To 6/86 ...
	R12
	38.00
	Mineral
	8.50

	1985
	All Engines
	R12
	38.00
	Mineral
	8.50

	
	ALTIMA
	

	2002-2004
	All Engines
	R134a
	18.00
	PAG 100
	5.00

	2001
	All Engines
	R134a
	22.00
	PAG 100
	6.75

	1993-2000
	All Engines
	R134a
	26.50
	PAG 100
	6.75

	
	AXXESS
	

	1990
	All Engines
	R12
	38.50
	Mineral
	6.75

	
	D21 PICKUP
	

	1993
	All Engines
	R134a
	30.00
	PAG 100
	6.75

	1990-1992
	All Engines
	R12
	32.00
	Mineral
	6.75

	1988-1989
	All Engines
	R12
	32.00
	Mineral
	7.00

	1987
	All Engines
	R12
	38.50
	Mineral
	6.75

	1986
	All Engines From 6/86 ...
	R12
	38.00
	Mineral
	6.75

	
	FRONTIER
	

	1998-2004
	All Engines
	R134a
	25.00
	PAG 100
	6.75

	
	MAXIMA
	

	2002-2003
	All Engines
	R134a
	18.00
	PAG 100
	6.00

	1997-2001
	All Engines
	R134a
	25.00
	PAG 100
	6.75

	1996
	All Engines
	R134a
	25.00
	PAG 100
	8.50

	1995
	All Engines
	R134a
	29.00
	PAG 100
	8.50

	1993-1994
	All Engines
	R134a
	33.50
	PAG 100
	6.75

	1991-1992
	All Engines
	R12
	33.50
	Mineral
	6.75

	1989-1990
	All Engines
	R12
	35.00
	Mineral
	6.75

	1986-1988
	All Engines
	R12
	38.00
	Mineral
	5.00

	1985
	All Engines
	R12
	38.00
	Mineral
	9.00

	
	MURANO
	

	2003-2004
	All Engines
	R134a
	19.00
	PAG 100
	6.00

	
	PATHFINDER
	

	2002-2004
	All Engines
	R134a
	16.00
	PAG 46
	6.00

	1996-2001
	All Engines
	R134a
	25.00
	PAG 100
	6.75

	1993-1995
	All Engines
	R134a
	30.00
	PAG 100
	6.75

	1989-1992
	All Engines
	R12
	32.00
	Mineral
	6.75

	
	PICKUP
	

	1993-1997
	All Engines
	R134a
	30.00
	PAG 100
	6.75

	1989-1992
	All Engines
	R12
	32.00
	Mineral
	6.75

	
	PULSAR NX
	

	1991-1993
	All Engines
	R12
	26.50
	Mineral
	6.75

	1987
	All Engines
	R12
	38.00
	Mineral
	6.75

	
	QUEST
	

	2004
	All Engines
	R134a
	32.00
	PAG 100
	8.50

	2001-2002
	All Engines w/ Front & Rear AC ...
	R134a
	54.00
	PAG 100
	11.00

	
	All Engines w/ Front AC Only ...
	R134a
	32.00
	PAG 100
	7.00

	1994-2000
	All Engines w/ Front & Rear AC ...
	R134a
	52.00
	PAG 100
	11.00

	
	All Engines w/ Front AC Only ...
	R134a
	32.00
	PAG 100
	7.00

	1993
	All Engines w/ Front & Rear AC ...
	R12
	56.00
	Mineral
	10.00

	
	All Engines w/ Front AC Only ...
	R12
	36.00
	Mineral
	7.00

	
	SENTRA
	

	2000-2004
	All Engines
	R134a
	18.00
	PAG 100
	6.00

	1994-1999
	All Engines
	R134a
	25.00
	PAG 100
	6.75

	1991-1993
	All Engines
	R12
	26.50
	Mineral
	6.75

	1989-1990
	All Engines
	R12
	33.50
	Mineral
	6.75

	1988
	All Engines
	R12
	38.00
	Mineral
	7.00

	1987
	All Engines
	R12
	38.00
	Mineral
	6.75

	1986
	All Engines
	R12
	35.00
	Mineral
	5.00

	1985
	All Engines
	R12
	35.00
	Mineral
	5.10

	
	STANZA
	

	1991-1992
	All Engines
	R12
	30.00
	Mineral
	6.75

	1990
	All Engines
	R12
	32.00
	Mineral
	6.75

	1989
	All Engines
	R12
	38.50
	Mineral
	6.75

	1988
	All Engines
	R12
	38.00
	Mineral
	7.00

	1987
	Coupe Sedan ...
	R12
	38.50
	Mineral
	6.75

	
	Wagon ...
	R12
	38.50
	Mineral
	5.00

	
	TITAN
	

	2004
	All Engines
	R134a
	24.7 - 26.5
	PAG 100
	7.00

	
	XTERRA
	

	2000-2004
	All Engines
	R134a
	25.00
	PAG 100
	7.00


Oldsmobile
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	98 / 98 REGENCY
	

	1996-1999
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1994-1995
	All Engines
	R134a
	39.00
	PAG 150
	8.00

	1991-1993
	All Engines
	R12
	29.00
	Mineral
	8.00

	1989-1990
	All Engines
	R12
	38.50
	Mineral
	8.00

	1988
	All Engines
	R12
	38.00
	Mineral
	8.00

	1985-1987
	All Engines
	R12
	46.00
	Mineral
	8.00

	
	ACHIEVA
	

	1995-1998
	All Engines
	R134a
	36.00
	PAG 150
	9.00

	1994
	All Engines
	R134a
	36.00
	PAG 150
	8.00

	1992-1993
	All Engines
	R12
	42.00
	Mineral
	8.00

	
	ALERO
	

	2002-2004
	All Engines
	R134a
	22.00
	PAG 150
	9.00

	1999-2001
	All Engines
	R134a
	28.00
	PAG 150
	9.00

	
	AURORA
	

	2003
	All Engines
	R134a
	35.00
	PAG 46
	9.00

	2001-2002
	All Engines
	R134a
	35.00
	PAG 150
	9.00

	1995-1999
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	
	BRAVADA
	

	2002-2004
	All Engines w/ Front AC Only ...
	R134a
	29.00
	PAG 46
	7.50

	
	All Engines w/ Front & Rear AC ...
	R134a
	48.00
	PAG 46
	8.50

	1999-2001
	All Engines
	R134a
	31.00
	PAG 150
	8.50

	1996-1998
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1994
	All Engines
	R134a
	40.00
	PAG 150
	8.00

	1993
	All Engines
	R12
	37.00
	Mineral
	8.00

	1991-1992
	All Engines
	R12
	40.00
	Mineral
	8.00

	
	CALAIS / CUTLASS CALAIS
	

	1987-1991
	All Engines
	R12
	36.00
	Mineral
	8.00

	1986
	All Engines
	R12
	36.00
	Mineral
	7.00

	
	CIERA / CUTLASS CIERA
	

	1994-1996
	4 Cyl. 2.2 Eng. ..
	R134a
	28.00
	PAG 150
	9.00

	
	6 Cyl. 3.1 Eng. ..
	R134a
	32.00
	PAG 150
	9.00

	1992-1993
	All Engines
	R12
	38.00
	Mineral
	9.00

	1985-1991
	All Engines
	R12
	44.00
	Mineral
	8.00

	
	CUTLASS / CUTLASS SUPREME
	

	1998-1999
	All Engines
	R134a
	28.00
	PAG 150
	9.00

	1997
	All Engines
	R134a
	30.00
	PAG 150
	8.00

	1994-1996
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1992-1993
	All Engines
	R12
	36.00
	Mineral
	9.00

	1989-1991
	All Engines
	R12
	36.00
	Mineral
	8.00

	1988
	All Engines
	R12
	56.00
	Mineral
	8.00

	1986-1987
	All Engines
	R12
	48.00
	Mineral
	8.00

	1985
	All Engines
	R12
	52.00
	Mineral
	8.00

	
	DELTA 88 CUSTOM CRUISER / CUSTOM CRUISER
	

	1991-1992
	All Engines
	R12
	49.50
	Mineral
	6.00

	1989-1990
	All Engines
	R12
	56.00
	Mineral
	6.00

	
	DELTA 88 SEDAN
	

	1998-1999
	All Engines
	R134a
	32.00
	PAG 150
	9.00

	1996-1997
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1995
	All Engines
	R134a
	39.00
	PAG 150
	8.00

	1994
	All Engines
	R12
	39.00
	Mineral
	8.00

	1992-1993
	All Engines
	R12
	36.00
	Mineral
	8.00

	1990-1991
	All Engines
	R12
	38.00
	Mineral
	6.00

	1989
	All Engines
	R12
	46.00
	Mineral
	6.00

	1988
	All Engines
	R12
	53.00
	Mineral
	6.00

	1986-1987
	All Engines
	R12
	38.00
	Mineral
	7.00

	1985
	All Engines
	R12
	46.00
	Mineral
	7.00

	
	INTRIGUE
	

	1998-2002
	All Engines
	R134a
	36.00
	PAG 150
	9.00

	
	LSS
	

	1996-1999
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	
	SILHOUETTE
	

	2001-2004
	All Engines w/ Front AC Only ...
	R134a
	27.00
	PAG 46
	7.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	35.00
	PAG 46
	8.00

	1998-2000
	All Engines w/ Front AC Only ...
	R134a
	32.00
	PAG 150
	8.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	46.00
	PAG 150
	11.00

	1994-1996
	All Engines w/ Front AC Only ...
	R134a
	36.00
	PAG 150
	8.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	48.00
	PAG 150
	11.00

	1993
	All Engines w/ Front AC Only ...
	R134a
	42.00
	PAG 150
	8.00

	
	All Engines w/ Front & Rear AC ...
	R134a
	56.00
	PAG 150
	11.00

	1992
	All Engines w/ Front AC Only ...
	R12
	51.00
	Mineral
	8.00

	
	All Engines w/ Front & Rear AC ...
	R12
	67.00
	Mineral
	11.00

	1991
	All Engines w/ Front AC Only ...
	R12
	51.00
	Mineral
	8.00

	1990
	All Engines w/ Front AC Only ...
	R12
	44.00
	Mineral
	8.00

	
	TORONADO
	

	1989-1992
	All Engines
	R12
	38.00
	Mineral
	8.00

	
	Trofeo ...
	R12
	38.00
	Mineral
	8.00


Peugeot
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	405
	

	1989-1991
	All Engines
	R12
	53.00
	Mineral
	6.75

	
	505
	

	1989-1991
	All Engines
	R12
	35.00
	Mineral
	6.75


Plymouth
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	ACCLAIM
	

	1994-1995
	All Engines
	R134a
	26.00
	PAG 46
	7.25

	1991-1993
	All Engines
	R12
	32.00
	Mineral
	7.25

	1989-1990
	All Engines
	R12
	38.00
	Mineral
	7.25

	
	BREEZE
	

	2000
	All Engines
	R134a
	19.00
	PAG 46
	5.00

	1999
	All Engines
	R134a
	20.00
	PAG 46
	5.00

	1998
	All Engines
	R134a
	26.00
	PAG 46
	5.00

	1996-1997
	All Engines
	R134a
	28.00
	PAG 46
	7.00

	
	CARAVELLE
	

	1986-1988
	All Engines
	R12
	38.00
	Mineral
	7.25

	
	HORIZON
	

	1988
	All Engines
	R12
	38.00
	Mineral
	7.25

	1987
	All Engines
	R12
	34.00
	Mineral
	7.25

	
	LASER
	

	1994
	All Engines
	R134a
	26.00
	PAG 46
	7.25

	1991-1993
	All Engines
	R12
	32.00
	Mineral
	7.25

	1990
	All Engines
	R12
	38.00
	Mineral
	7.25

	
	NEON
	

	2000-2001
	All Engines
	R134a
	27.00
	PAG 46
	6.00

	1999
	All Engines
	R134a
	28.00
	PAG 46
	4.75

	1997-1998
	All Engines
	R134a
	25.00
	PAG 46
	6.75

	1995-1996
	All Engines
	R134a
	29.00
	PAG 46
	4.75

	
	PROWLER
	

	2000
	All Engines
	R134a
	21.00
	PAG 46
	5.00

	1997-1999
	All Engines
	R134a
	24.00
	PAG 46
	5.00

	
	RELIANT
	

	1986-1988
	All Engines
	R12
	38.00
	Mineral
	7.25

	1985
	All Engines
	R12
	37.00
	Mineral
	7.25

	
	SUNDANCE
	

	1991-1993
	All Engines
	R12
	32.00
	Mineral
	7.25

	1987-1988
	All Engines
	R12
	38.00
	Mineral
	7.25

	
	VOYAGER / GRAND VOYAGER
	

	1996-2001
	All Engines w/ Front & Rear AC ...
	R134a
	46.00
	PAG 46
	13.00

	
	All Engines w/ Front AC Only ...
	R134a
	34.00
	PAG 46
	9.00

	1994-1995
	All Engines w/ Front & Rear AC ...
	R134a
	52.00
	PAG 46
	13.00

	
	All Engines w/ Front AC Only ...
	R134a
	36.00
	PAG 46
	6.75

	1993
	All Engines w/ Front & Rear AC ...
	R134a
	50.00
	PAG 46
	13.00

	
	All Engines w/ Front AC Only ...
	R134a
	36.00
	PAG 46
	6.75

	1992
	All Engines w/ Front & Rear AC - w/ Fixed Displacement Compressor ...
	R12
	44.00
	Mineral
	13.00

	
	All Engines w/ Front AC Only - w/ Fixed Displacement Compressor ...
	R12
	33.50
	Mineral
	7.25

	
	All Engines w/ Front & Rear AC - w/ Variable Displacement Compressor
	R12
	44.00
	Mineral
	13.00

	
	All Engines w/ Front AC Only - w/ Variable Displacement Compressor ...
	R12
	33.50
	Mineral
	8.75

	1990-1991
	All Engines w/ Front & Rear AC - w/ Fixed Displacement Compressor ...
	R12
	43.00
	Mineral
	13.00

	
	All Engines w/ Front AC Only - w/ Fixed Displacement Compressor ...
	R12
	32.00
	Mineral
	7.25

	
	All Engines w/ Front & Rear AC - w/ Variable Displacement Compressor
	R12
	43.00
	Mineral
	13.00

	
	All Engines w/ Front AC Only - w/ Variable Displacement Compressor ...
	R12
	32.00
	Mineral
	8.75

	1989
	All Engines w/ Front & Rear AC - w/ Fixed Displacement Compressor ...
	R12
	43.00
	Mineral
	13.00

	
	All Engines w/ Front AC Only - w/ Fixed Displacement Compressor ...
	R12
	38.00
	Mineral
	7.25

	
	All Engines w/ Front & Rear AC - w/ Variable Displacement Compressor
	R12
	43.00
	Mineral
	13.00

	
	All Engines w/ Front AC Only - w/ Variable Displacement Compressor ...
	R12
	38.00
	Mineral
	8.75

	1988
	All Engines w/ Front AC Only ...
	R12
	44.00
	Mineral
	7.25

	1986-1987
	All Engines w/ Front AC Only ...
	R12
	38.00
	Mineral
	7.00


Pontiac
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	6000
	

	1988-1991
	All Engines
	R12
	44.00
	Mineral
	8.00

	1986-1987
	Excl. STE ...
	R12
	44.00
	Mineral
	8.00

	
	STE ...
	R12
	36.00
	Mineral
	8.00

	1985
	All Engines
	R12
	44.00
	Mineral
	8.00

	
	AZTEK
	

	2001-2004
	All Engines
	R134a
	27.00
	PAG 46
	7.50

	
	BONNEVILLE
	

	2002-2004
	All Engines
	R134a
	35.00
	PAG 150
	9.00

	1998-2001
	All Engines
	R134a
	32.00
	PAG 150
	9.00

	1996-1997
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1994-1995
	All Engines
	R134a
	39.00
	PAG 150
	8.00

	1991-1993
	All Engines
	R12
	39.00
	Mineral
	8.00

	1989-1990
	All Engines
	R12
	38.50
	Mineral
	8.00

	1987-1988
	All Engines
	R12
	53.00
	Mineral
	8.00

	
	FIERO
	

	1986-1988
	All Engines
	R12
	40.00
	Mineral
	8.00

	
	FIREBIRD
	

	1998-2002
	All Engines
	R134a
	24.00
	PAG 150
	9.00

	1997
	6 Cyl. 3.8 Eng. ..
	R134a
	26.00
	PAG 150
	8.00

	
	8 Cyl. 5.7 Eng. ..
	R134a
	32.00
	PAG 150
	8.00

	1996
	6 Cyl. 3.8 Eng. ..
	R134a
	28.00
	PAG 150
	8.00

	
	8 Cyl. 5.7 Eng. ..
	R134a
	32.00
	PAG 150
	8.00

	1993-1995
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1989-1992
	All Engines
	R12
	36.00
	Mineral
	6.00

	1986
	All Engines
	R12
	48.00
	Mineral
	8.00

	
	GRAND AM
	

	2002-2004
	All Engines
	R134a
	22.00
	PAG 150
	9.00

	1999-2001
	All Engines
	R134a
	28.00
	PAG 150
	8.00

	1995-1998
	All Engines
	R134a
	36.00
	PAG 150
	9.00

	1994
	All Engines
	R134a
	36.00
	PAG 150
	8.00

	1993
	All Engines
	R12
	42.00
	Mineral
	8.00

	1992
	All Engines
	R12
	42.50
	Mineral
	8.00

	1988-1991
	All Engines
	R12
	36.00
	Mineral
	8.00

	1986-1987
	All Engines
	R12
	44.00
	Mineral
	8.00

	
	GRAND PRIX
	

	2004
	All Engines
	R134a
	37.00
	PAG 150
	8.00

	1998-2003
	All Engines
	R134a
	36.00
	PAG 150
	9.00

	1997
	All Engines
	R134a
	36.00
	PAG 150
	8.00

	1994-1996
	All Engines
	R134a
	32.00
	PAG 150
	8.00

	1992-1993
	All Engines
	R12
	36.00
	Mineral
	9.00

	1988-1991
	All Engines
	R12
	36.00
	Mineral
	8.00

	
	LEMANS
	

	1988-1993
	All Engines
	R12
	35.00
	Mineral
	8.00

	
	MONTANA
	

	2001-2004
	All Engines w/ Front & Rear AC ...
	R134a
	35.00
	PAG 46
	8.00

	
	All Engines w/ Front AC ...
	R134a
	27.00
	PAG 46
	7.00

	1999-2000
	All Engines w/ Front & Rear AC ...
	R134a
	46.00
	PAG 150
	11.00

	
	All Engines w/ Front AC ...
	R134a
	32.00
	PAG 150
	8.00

	
	PARISIENNE
	

	1985-1986
	All Engines
	R12
	60.00
	Mineral
	8.00

	
	SUNBIRD
	

	1994
	All Engines
	R134a
	36.00
	PAG 150
	9.00

	1988-1993
	All Engines
	R12
	36.00
	Mineral
	8.00

	1987
	All Engines
	R12
	44.00
	Mineral
	8.00

	
	SUNFIRE
	

	1995-2002
	All Engines
	R134a
	24.00
	PAG 150
	9.00

	
	TRANS SPORT
	

	1997-1998
	All Engines w/ Single Unit ...
	R134a
	32.00
	PAG 150
	8.00

	
	All Engines w/ Dual Unit ...
	R134a
	46.00
	PAG 150
	11.00

	1995
	All Engines w/ Single Unit ...
	R134a
	36.00
	PAG 150
	8.00

	
	All Engines w/ Dual Unit ...
	R134a
	48.00
	PAG 150
	11.00

	
	VIBE
	

	2003
	All Engines
	R134a
	23.00
	PAG 150
	8.00


Porsche
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	911 SERIES
	

	2001-2004
	All Engines
	R134a
	31.00
	PAG 46
	6.50

	1999-2000
	All Engines
	R134a
	28.00
	PAG 46
	6.50

	1993-1998
	All Engines
	R134a
	29.50
	PAG 46
	4.75

	1990-1992
	All Engines
	R12
	33.00
	Mineral
	3.50

	1989
	All Engines
	R12
	33.00
	Mineral
	4.00

	
	924 SERIES
	

	1987-1988
	All Engines
	R12
	30.00
	Mineral
	7.25

	
	928 SERIES
	

	1993-1995
	All Engines
	R134a
	30.00
	PAG 46
	9.00

	1989-1991
	All Engines
	R12
	33.50
	Mineral
	9.00

	
	944 SERIES
	

	1989-1991
	All Engines
	R12
	33.50
	Mineral
	5.75

	
	968 SERIES
	

	1993-1995
	All Engines
	R134a
	30.00
	PAG 46
	4.00

	1992
	All Engines
	R12
	33.50
	Mineral
	5.75

	
	BOXSTER
	

	1997-2004
	All Engines
	R134a
	30.00
	PAG 46
	6.50


Saab
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	9-3
	

	1999-2001
	All Engines
	R134a
	28.00
	PAG 46
	5.00

	
	9-5
	

	1999-2001
	All Engines
	R134a
	31.00
	PAG 100
	5.00

	
	900 SERIES
	

	1997-1998
	900 - w/ Sanden Compressor ...
	R134a
	25.50
	PAG 100
	5.00

	
	900 - w/ Seiko Seiki Compressor ...
	R134a
	25.50
	PAG 100
	6.75

	1994-1996
	900 ...
	R134a
	25.50
	PAG 100
	6.75

	1989-1992
	900 ...
	R12
	35.00
	Mineral
	5.75

	
	9000 SERIES
	

	1992
	9000 w/ Front AC ...
	R12
	39.00
	Mineral
	5.50

	
	9000 w/ Front & Rear AC ...
	R12
	48.00
	Mineral
	6.75

	1989-1991
	9000 ...
	R12
	39.00
	Mineral
	5.50


Saturn
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	ION
	

	2003-2004
	All Engines
	R134a
	14.50
	--
	5.00

	
	L SERIES
	

	2001-2004
	All Engines
	R134a
	33.50
	PAG 100
	7.50

	
	LS SERIES
	

	2000
	All Engines
	R134a
	33.50
	PAG 100
	7.50

	
	LW SERIES
	

	2001-2004
	All Engines
	R134a
	33.50
	PAG 100
	7.50

	
	SC SERIES
	

	1997-2002
	All Engines
	R134a
	24.00
	PAG 100
	5.00

	1994-1996
	All Engines
	R134a
	24.00
	PAG 100
	6.75

	1991-1993
	All Engines
	R12
	36.00
	Mineral
	6.75

	
	SL SERIES
	

	1997-2002
	All Engines
	R134a
	24.00
	PAG 100
	5.00

	1994-1996
	All Engines
	R134a
	24.00
	PAG 100
	6.75

	1991-1993
	All Engines
	R12
	36.00
	Mineral
	6.75

	
	SW SERIES
	

	1997-2001
	All Engines
	R134a
	24.00
	PAG 100
	5.00

	1994-1996
	All Engines
	R134a
	24.00
	PAG 100
	6.75

	1993
	All Engines
	R12
	36.00
	Mineral
	6.75

	
	VUE
	

	2002-2004
	2.2 Eng. ..
	R134a
	24.00
	PAG 100
	7.00

	
	2.4 Eng. ..
	R134a
	28.00
	PAG 100
	7.00


Sterling
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	827
	

	1990-1991
	All Engines
	R12
	36.00
	Mineral
	5.00


Subaru
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	FORESTER
	

	2001
	All Engines
	R134a
	22.00
	PAG 100
	6.00

	2000
	All Engines
	R134a
	23.00
	PAG 100
	5.25

	1998-1999
	All Engines
	R134a
	21.00
	PAG 100
	5.25

	
	IMPREZA
	

	1997-2000
	All Engines
	R134a
	21.00
	PAG 100
	5.00

	1994-1996
	All Engines
	R134a
	24.00
	PAG 100
	4.00

	1993
	All Engines
	R12
	26.50
	Mineral
	6.00

	
	IMPREZA OUTBACK
	

	2000
	All Engines
	R134a
	24.00
	PAG 100
	5.50

	
	LEGACY
	

	2001
	All Engines
	R134a
	23.00
	PAG 100
	4.50

	1999-2000
	All Engines
	R134a
	23.00
	PAG 100
	5.50

	1994-1998
	All Engines
	R134a
	26.50
	PAG 100
	5.50

	1992-1993
	All Engines - w/ Calsonic Compressor...
	R12
	24.00
	Mineral
	8.00

	
	All Engines - w/ Zexel Compressor ...
	R12
	24.00
	Mineral
	5.00

	1990-1991
	All Engines - w/ Calsonic Compressor...
	R12
	24.00
	Mineral
	8.00

	
	All Engines - w/ Diesel Kiki Compresor...
	R12
	24.00
	Mineral
	5.00

	
	LEGACY OUTBACK
	

	2000
	All Engines
	R134a
	24.00
	PAG 100
	5.50

	1996-1998
	All Engines
	R134a
	24.00
	PAG 100
	5.25

	1995
	All Engines
	R134a
	24.00
	PAG 100
	6.00

	
	LOYALE
	

	1990-1993
	All Engines From 3/92 ...
	R12
	30.00
	Mineral
	4.75

	
	All Engines To 3/92 ...
	R12
	28.00
	Mineral
	5.00

	
	OUTBACK
	

	2001
	All Engines
	R134a
	25.00
	PAG 100
	4.00

	
	SEDAN, COUPE, WAGON
	

	1989
	HL/GL/GL10 - w/ Hitachi Compressor...
	R12
	28.00
	Mineral
	5.00

	
	HL/GL/GL10 - w/ Panasonic Compressor ...
	R12
	30.00
	Mineral
	4.75

	1985-1988
	All Engines
	R12
	28.00
	Mineral
	4.00

	
	SVX
	

	1993-1996
	All Engines
	R134a
	23.00
	PAG 100
	5.00

	1992
	All Engines
	R12
	23.00
	Mineral
	5.00

	
	XT
	

	1989-1991
	All Engines
	R12
	28.00
	Mineral
	5.00


Suzuki
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	ESTEEM
	

	1996-2000
	All Engines
	R134a
	21.00
	--
	5.00

	
	GRAND VITARA
	

	2001
	All Engines
	R134a
	14.00
	--
	4.00

	1999-2000
	All Engines
	R134a
	21.00
	--
	4.00

	
	SAMURAI
	

	1994-1995
	All Engines
	R134a
	18.00
	PAG 46
	3.50

	1989-1993
	All Engines
	R12
	18.00
	Mineral
	5.00

	1987-1988
	All Engines
	R12
	28.00
	Mineral
	4.00

	
	SIDEKICK
	

	1994-1998
	All Engines
	R134a
	21.00
	PAG 46
	6.00

	1989-1993
	All Engines
	R12
	21.00
	Mineral
	5.00

	
	SWIFT
	

	2001
	All Engines
	R134a
	18.00
	--
	3.50

	1994-2000
	All Engines
	R134a
	18.00
	PAG 46
	3.50

	1989-1993
	All Engines
	R12
	18.00
	Mineral
	5.00

	
	VITARA
	

	1999-2002
	All Engines
	R134a
	16.00
	--
	4.00

	
	X-90
	

	1996-1998
	All Engines
	R134a
	21.00
	--
	6.00


Toyota
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	4RUNNER
	

	2001-2002
	All Engines
	R134a
	25.00
	PAG 46
	7.00

	1996-2000
	All Engines
	R134a
	23.00
	PAG 46
	7.00

	1994-1995
	All Engines
	R134a
	25.00
	PAG 46
	7.00

	1991-1993
	All Engines
	R12
	30.00
	Mineral
	7.00

	1989-1990
	All Engines
	R12
	29.00
	Mineral
	7.75

	
	AVALON
	

	2001
	All Engines
	R134a
	23.00
	PAG 46
	7.00

	1995-2000
	All Engines
	R134a
	30.00
	PAG 46
	7.00

	
	CAMRY
	

	2002-2003
	All Engines
	R134a
	19.00
	PAG 46
	8.00

	2001
	All Engines
	R134a
	30.00
	PAG 46
	8.00

	1997-2000
	All Engines
	R134a
	28.00
	PAG 46
	8.00

	1994-1996
	All Engines
	R134a
	30.00
	PAG 46
	8.00

	1993
	All Engines
	R12
	33.50
	Mineral
	7.25

	1992
	All Engines
	R12
	35.00
	Mineral
	7.25

	1991
	All Engines
	R12
	27.00
	Mineral
	7.00

	1989-1990
	All Engines
	R12
	27.00
	Mineral
	7.50

	1987-1988
	All Engines
	R12
	27.00
	Mineral
	8.75

	1985-1986
	All Engines
	R12
	26.00
	Mineral
	8.75

	
	CELICA
	

	2000-2004
	All Engines
	R134a
	16.00
	PAG 46
	7.00

	1994-1999
	All Engines
	R134a
	23.00
	PAG 46
	7.00

	1993
	All Engines
	R12
	25.50
	Mineral
	7.00

	1991-1992
	All Engines
	R12
	27.00
	Mineral
	7.00

	1989-1990
	All Engines
	R12
	27.00
	Mineral
	7.75

	1987-1988
	All Engines
	R12
	27.00
	Mineral
	8.75

	1985
	All Engines
	R12
	26.00
	Mineral
	8.75

	
	COROLLA
	

	2003-2004
	All Engines
	R134a
	18.00
	PAG 46
	7.00

	2001-2002
	All Engines
	R134a
	24.00
	PAG 46
	7.00

	1999-2000
	All Engines
	R134a
	23.00
	PAG 46
	7.00

	1996-1998
	All Engines
	R134a
	24.00
	PAG 46
	7.00

	1994-1995
	All Engines
	R134a
	26.50
	PAG 46
	7.00

	1993
	All Engines
	R12
	26.50
	Mineral
	7.00

	1991-1992
	All Engines
	R12
	27.00
	Mineral
	7.00

	1989-1990
	All Engines
	R12
	27.00
	Mineral
	7.75

	1988
	All Engines
	R12
	27.00
	Mineral
	7.50

	1987
	All Engines
	R12
	26.00
	Mineral
	8.75

	1986
	All Engines
	R12
	27.00
	Mineral
	8.75

	1985
	All Engines
	R12
	26.00
	Mineral
	8.75

	
	CRESSIDA
	

	1989-1992
	All Engines
	R12
	30.00
	Mineral
	7.00

	1986
	All Engines
	R12
	26.00
	Mineral
	8.75

	
	ECHO
	

	2000-2004
	All Engines
	R134a
	16.00
	PAG 46
	7.75

	
	HIGHLANDER
	

	2001-2004
	All Engines
	R134a
	24.00
	PAG 46
	7.75

	
	LAND CRUISER
	

	2001-2004
	All Engines w/ Front AC ...
	R134a
	24.00
	PAG 46
	--

	
	All Engines w/ Front & Rear AC ...
	R134a
	38.00
	PAG 46
	--

	1998-2000
	All Engines w/ Front & Rear AC ...
	R134a
	39.00
	PAG 46
	8.50

	
	All Engines w/ Front AC ...
	R134a
	28.00
	PAG 46
	7.25

	1994-1997
	All Engines
	R134a
	30.00
	PAG 46
	7.00

	1993
	All Engines
	R12
	32.00
	Mineral
	7.00

	1991-1992
	All Engines
	R12
	33.50
	Mineral
	7.00

	1989-1990
	All Engines
	R12
	32.00
	Mineral
	7.75

	
	MATRIX
	

	2003-2004
	All Engines
	R134a
	18.00
	PAG 46
	--

	
	MR2
	

	2000-2004
	All Engines
	R134a
	18.00
	PAG 46
	--

	1994-1995
	All Engines
	R134a
	25.50
	PAG 46
	7.00

	1993
	All Engines
	R12
	30.00
	Mineral
	7.25

	1991-1992
	All Engines
	R12
	32.00
	Mineral
	7.25

	1989
	All Engines
	R12
	29.00
	Mineral
	8.75

	
	PASEO
	

	1996-1997
	All Engines
	R134a
	21.00
	PAG 46
	7.00

	1994-1995
	All Engines
	R134a
	25.00
	PAG 46
	7.00

	1993
	All Engines
	R12
	26.50
	Mineral
	7.00

	1992
	All Engines
	R12
	28.00
	Mineral
	7.00

	
	PICKUP - STANDARD
	

	1995
	All Engines
	R134a
	20.00
	PAG 46
	7.00

	1994
	All Engines
	R134a
	26.50
	PAG 46
	7.00

	1993
	All Engines
	R12
	29.00
	Mineral
	7.00

	1991-1992
	All Engines
	R12
	27.00
	Mineral
	7.00

	1989-1990
	All Engines
	R12
	29.00
	Mineral
	7.00

	1987-1988
	All Engines
	R12
	27.00
	Mineral
	7.50

	1985-1986
	All Engines
	R12
	26.00
	Mineral
	7.50

	
	PREVIA
	

	1994-1997
	All Engines w/ Front & Rear AC ...
	R134a
	41.00
	PAG 46
	7.00

	
	All Engines w/ Front AC ...
	R134a
	32.00
	PAG 46
	7.00

	1993
	All Engines w/ Front & Rear AC ...
	R12
	42.50
	Mineral
	7.75

	
	All Engines w/ Front AC ...
	R12
	33.50
	Mineral
	7.75

	1991-1992
	All Engines w/ Front & Rear AC ...
	R12
	44.00
	Mineral
	7.75

	
	All Engines w/ Front AC ...
	R12
	35.00
	Mineral
	7.75

	
	RAV4
	

	2001-2004
	All Engines
	R134a
	19.00
	PAG 46
	7.00

	1996-2000
	All Engines
	R134a
	25.00
	PAG 46
	7.00

	
	SEQUOIA
	

	2001-2004
	All Engines w/ Front AC Only ...
	R134a
	28.00
	PAG 46
	--

	
	All Engines w/ Front & Rear AC ...
	R134a
	39.00
	PAG 46
	--

	
	SIENNA
	

	2001-2003
	All Engines
	R134a
	38.00
	PAG 46
	7.50

	1998-2000
	All Engines
	R134a
	28.00
	PAG 46
	7.25

	
	SOLARA
	

	2001-2003
	All Engines
	R134a
	28.00
	PAG 46
	7.50

	1999-2000
	All Engines
	R134a
	28.00
	PAG 46
	8.00

	
	SUPRA
	

	1996-1998
	All Engines
	R134a
	25.00
	PAG 46
	7.75

	1994-1995
	All Engines
	R134a
	25.00
	PAG 46
	7.00

	1989-1992
	All Engines
	R12
	27.00
	Mineral
	7.00

	1987
	All Engines
	R12
	26.00
	Mineral
	8.80

	
	T100
	

	1993-1998
	All Engines
	R134a
	23.00
	PAG 46
	7.75

	
	TACOMA
	

	2001
	All Engines
	R134a
	23.00
	PAG 46
	7.75

	1997-2000
	All Engines
	R134a
	21.00
	PAG 46
	7.75

	1995-1996
	All Engines
	R134a
	21.00
	PAG 46
	7.00

	
	TERCEL
	

	1998
	All Engines
	R134a
	16.00
	PAG 46
	7.00

	1995-1997
	All Engines
	R134a
	23.00
	PAG 46
	7.00

	1994
	All Engines
	R134a
	24.00
	PAG 46
	7.00

	1993
	All Engines
	R12
	26.50
	Mineral
	7.00

	1992
	All Engines
	R12
	28.00
	Mineral
	7.00

	1991
	All Engines
	R12
	26.50
	Mineral
	7.00

	1989-1990
	All Engines
	R12
	27.00
	Mineral
	7.75

	1987-1988
	All Engines
	R12
	27.00
	Mineral
	8.50

	1985-1986
	All Engines
	R12
	26.00
	Mineral
	8.50

	
	TUNDRA
	

	2000-2003
	All Engines
	R134a
	22.00
	PAG 46
	7.75

	
	VAN / VAN WAGON
	

	1986-1989
	All Engines w/ Front AC ...
	R12
	25.00
	Mineral
	5.00

	
	All Engines w/ Front & Rear AC ...
	R12
	50.00
	Mineral
	6.00


Volkswagen
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	BEETLE
	

	2001-2003
	All Engines
	R134a
	26.00
	PAG 46
	4.50

	1998-2000
	All Engines
	R134a
	24.00
	PAG 46
	4.50

	
	CABRIO
	

	2001-2002
	All Engines
	R134a
	27.00
	PAG 46
	4.50

	1995-2000
	All Engines
	R134a
	28.00
	PAG 46
	4.00

	
	CABRIOLET
	

	1986-1993
	All Engines
	R12
	32.00
	Mineral
	4.75

	
	CORRADO
	

	1994
	All Engines
	R134a
	35.00
	PAG 46
	4.00

	1993
	All Engines
	R12
	30.00
	Mineral
	5.00

	
	All Engines
	R134a
	35.00
	PAG 46
	4.00

	1990-1992
	All Engines
	R12
	30.00
	Mineral
	5.00

	
	EUROVAN
	

	1993-2001
	All Engines w/ Front & Rear AC ...
	R134a
	48.00
	PAG 150
	8.00

	
	All Engines w/ Front AC ...
	R134a
	33.50
	PAG 150
	4.50

	
	FOX
	

	1989-1993
	All Engines
	R12
	38.00
	Mineral
	6.00

	1988
	All Engines
	R12
	40.00
	Mineral
	--

	
	GOLF
	

	2001-2003
	All Engines
	R134a
	27.00
	PAG 46
	4.50

	1999-2000
	All Engines
	R134a
	26.00
	PAG 46
	4.50

	1993-1998
	All Engines
	R134a
	28.00
	PAG 46
	4.00

	1989-1992
	All Engines
	R12
	39.00
	Mineral
	4.50

	1986-1987
	4 Cyl. 1.8 Eng. ..
	R12
	40.00
	Mineral
	4.50

	
	JETTA
	

	2001-2003
	All Engines
	R134a
	27.00
	PAG 46
	4.50

	1999-2000
	All Engines
	R134a
	26.00
	PAG 46
	4.50

	1993-1998
	All Engines
	R134a
	28.00
	PAG 46
	4.00

	1989-1992
	All Engines
	R12
	39.00
	Mineral
	4.50

	1986-1988
	4 Cyl. 1.8 Eng. ..
	R12
	40.00
	Mineral
	4.50

	
	PASSAT
	

	1998-2003
	All Engines
	R134a
	23.00
	PAG 46
	8.50

	1995-1997
	All Engines
	R134a
	33.00
	PAG 46
	4.00

	1993-1994
	All Engines
	R134a
	40.00
	PAG 46
	4.00

	1990-1992
	All Engines
	R12
	40.00
	Mineral
	4.00

	
	SCIROCCO
	

	1989
	All Engines
	R12
	40.00
	Mineral
	4.75

	
	VANAGON
	

	1989-1991
	All Engines
	R12
	51.00
	Mineral
	8.00


Volvo
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	240 SERIES
	

	1993
	All Engines
	R134a
	26.50
	PAG 46
	7.50

	1991-1992
	All Engines
	R12
	39.00
	Mineral
	6.75

	1987-1990
	All Engines
	R12
	46.00
	Mineral
	6.75

	1986
	All Engines - w/ Diesel Kiki Compressor ...
	R12
	46.00
	Mineral
	6.75

	
	All Engines - w/ York Compressor ...
	R12
	46.00
	Mineral
	10.00

	1985
	4 Cyl. 2.3 Eng. ..
	R12
	46.00
	Mineral
	10.00

	
	740 SERIES
	

	1992
	All Engines - w/ SD5 Compressor ...
	R12
	40.00
	Mineral
	4.50

	
	All Engines - w/ SD7 Compressor ...
	R12
	40.00
	Mineral
	8.00

	
	All Engines - w/ Zexel Compressor ...
	R12
	40.00
	Mineral
	6.75

	1991
	All Engines - w/ SD5 Compressor ...
	R12
	39.00
	Mineral
	4.50

	
	All Engines - w/ SD7 Compressor ...
	R12
	39.00
	Mineral
	8.00

	
	All Engines - w/ Zexel Compressor ...
	R12
	39.00
	Mineral
	6.75

	1989-1990
	All Engines - w/ Diesel Kiki Compressor ...
	R12
	41.50
	Mineral
	6.75

	
	All Engines - w/ R4 Compressor ...
	R12
	41.50
	Mineral
	6.00

	
	All Engines - w/ SD5 Compressor ...
	R12
	41.50
	Mineral
	4.50

	
	All Engines - w/ SD7 Compressor ...
	R12
	41.50
	Mineral
	4.50

	
	All Engines - w/ York Compressor ...
	R12
	41.50
	Mineral
	10.00

	1987-1988
	All Engines
	R12
	46.00
	Mineral
	6.75

	1986
	4 Cyl. 2.3 Eng. - w/ Diesel Kiki Compressor ...
	R12
	46.00
	Mineral
	6.75

	
	4 Cyl. 2.3 Eng. - w/ York Compressor...
	R12
	46.00
	Mineral
	10.00

	
	760 SERIES
	

	1989-1990
	All Engines - w/ Diesel Kiki Compresor...
	R12
	41.50
	Mineral
	6.75

	
	All Engines - w/ R4 Compressor ...
	R12
	41.50
	Mineral
	6.00

	
	All Engines - w/ SD5 Compressor ...
	R12
	41.50
	Mineral
	4.50

	
	All Engines - w/ SD7 Compressor ...
	R12
	41.50
	Mineral
	4.50

	
	All Engines - w/ York Compressor ...
	R12
	41.50
	Mineral
	10.00

	
	780 SERIES
	

	1991
	All Engines - w/ SD5 Compressor ...
	R12
	39.00
	Mineral
	4.50

	
	All Engines - w/ SD7 Compressor ...
	R12
	39.00
	Mineral
	8.00

	
	All Engines - w/ Zexel Compressor ...
	R12
	39.00
	Mineral
	6.75

	1989-1990
	All Engines - w/ Diesel Kiki Compresor...
	R12
	41.50
	Mineral
	6.75

	
	All Engines - w/ R4 Compressor ...
	R12
	41.50
	Mineral
	6.00

	
	All Engines - w/ SD5 Compressor ...
	R12
	41.50
	Mineral
	4.50

	
	All Engines - w/ SD7 Compressor ...
	R12
	41.50
	Mineral
	4.50

	
	All Engines - w/ York Compressor ...
	R12
	41.50
	Mineral
	10.00

	
	850 SERIES
	

	1996-1997
	All Engines
	R134a
	36.00
	PAG 100
	6.75

	
	All Engines - w/ Seiko Seiki Compressor ...
	R134a
	32.00
	PAG 150
	5.50

	1993-1995
	All Engines
	R134a
	36.00
	PAG 100
	6.75

	
	940 SERIES
	

	1993-1995
	All Engines - w/ SD5 Compressor ...
	R134a
	33.50
	PAG 150
	4.50

	
	All Engines - w/ SD7 Compressor ...
	R134a
	33.50
	PAG 150
	8.00

	
	All Engines - w/ Seiko Seiki Compressor ...
	R134a
	33.50
	PAG 150
	7.50

	1992
	All Engines - w/ SD5 Compressor ...
	R12
	40.00
	Mineral
	4.50

	
	All Engines - w/ SD7 Compressor ...
	R12
	40.00
	Mineral
	8.00

	
	All Engines - w/ Zexel Compressor ...
	R12
	40.00
	Mineral
	6.75

	1991
	All Engines - w/ SD5 Compressor ...
	R12
	39.00
	Mineral
	4.50

	
	All Engines - w/ SD7 Compressor ...
	R12
	39.00
	Mineral
	8.00

	
	All Engines - w/ Zexel Compressor ...
	R12
	39.00
	Mineral
	6.75

	
	960 SERIES
	

	1996-1997
	All Engines - w/ SD7 Compressor ...
	R134a
	32.00
	PAG 150
	8.50

	1993-1995
	All Engines - w/ SD5 Compressor ...
	R134a
	32.00
	PAG 150
	4.50

	
	All Engines - w/ SD7 Compressor ...
	R134a
	32.00
	PAG 150
	8.00

	
	All Engines - w/ Seiko Seiki Compressor ...
	R134a
	32.00
	PAG 150
	6.75

	1992
	All Engines - w/ SD5 Compressor ...
	R12
	39.00
	Mineral
	4.50

	
	All Engines - w/ SD7 Compressor ...
	R12
	39.00
	Mineral
	8.00

	
	All Engines - w/ Zexel Compressor ...
	R12
	39.00
	Mineral
	6.75

	
	C70
	

	2002-2004
	All Engines
	R134a
	35.00
	PAG 100
	6.75

	1998-2001
	All Engines
	R134a
	29.00
	PAG 100
	6.75

	
	S40
	

	2001-2004
	All Engines
	R134a
	30.00
	PAG 100
	6.00

	2000
	All Engines
	R134a
	31.00
	PAG 100
	6.00

	
	S60
	

	2001-2004
	All Engines
	R134a
	35.00
	PAG 100
	6.00

	
	S70
	

	1998-2000
	All Engines
	R134a
	29.00
	PAG 150
	6.75

	
	S80
	

	2002-2004
	All Engines
	R134a
	35.00
	PAG 100
	6.75

	1999-2001
	All Engines
	R134a
	35.00
	PAG 100
	6.00

	
	S90
	

	1998
	All Engines
	R134a
	32.00
	PAG 150
	6.75

	
	V40
	

	2000-2004
	All Engines
	R134a
	32.00
	PAG 100
	6.00

	
	V70
	

	2001-2004
	All Engines
	R134a
	35.00
	PAG 100
	6.75

	1998-2000
	All Engines
	R134a
	29.00
	PAG 100
	6.75

	
	V90
	

	1998
	All Engines
	R134a
	32.00
	PAG 150
	6.75

	
	XC90
	

	2003-2004
	All Engines w/ Front & Rear AC ...
	R134a
	46.00
	PAG 100
	7.00

	
	All Engines w/ Front AC Only ...
	R134a
	35.00
	PAG 100
	6.00


Yugo
	Year
	Model
	Refrigerant
	Oil

	
	
	Type
	Ounces
	Type
	Ounces

	
	GV
	

	1989-1991
	All Engines
	R12
	32.00
	Mineral
	6.00

	
	GVX
	

	1989
	All Engines
	R12
	32.00
	Mineral
	6.00


