USB-RS232 Driver Assignment and Installation for Reefer Manager

- If your computer has both a 9-pin serial port (COM Port) and a USB port, we recommend that you use the COM Port for communications.  These instructions are intended for those computers that do not have a 9-pin COM port.

- Most computers have more than one USB port.  Whichever port you connect the adapter to during this procedure is the only port that will operate following this procedure.  If it is a requirement that more than one of the ports be available, this procedure must be followed for each port.

- Unplug any USB Serial Adapter if connected.

- Install CCN, USB-RS232, ReeferManager, Reports, in that order, from the version 03.08.00 CD. Find autorun.exe on the CD in order to bring up the Installation menu. Follow onscreen instructions.  Exit install screen and remove CD.

- Insert CD that comes with the USB Adapter, and follow the on-screen instructions to install the drivers. Exit out when complete and remove the CD.

- Connect the USB Serial Adapter, if any windows installation instructions pop-up at this point, then the installation of the USB adapter did not install properly. Follow the on-screen instructions to re-install the drivers.

- Click on the the Start Button, then go to “All Programs” then “Carrier” and click on DriverConfig.

- With Serial Adapter plugged in, the USB - Serial Adapter driver should be visible in the DriverConfig Utility dropdown menu as shown below.

- NOTE:  Your Driver Configuration list will appear differently than the one shown above, depending on what drivers are already installed on your computer.

- Select the Appropriate USB – Serial Adapter from the list.

  - Click “Install” button on the Driver Configuration menu.

  - In the pop-up box, click the down arrow next to the “Look In” field, and scroll to C:\Windows\System32\.  Locate and highlight file vspDevUpr.sys, then Click Open button. This will install driver and will show Message “Driver Installed Successfully”.

  - Click “Configure” button in the Driver Configuration pop-up box.

  - Unplug the USB Serial Adapter, wait a couple seconds and plug it again. Click on Test Button. The Message “Driver has loaded successfully” will appear if everything has been done correctly. Now the user will be able use this Adapter through Reefer Manager Application.

- Open Reefer manager and select PC Setup from the drop-down menu across the top.

- In the PC Set Up screen, you should see a COM Radio button (may not be COM4) if Driver were installed and configured properly. Select the COM Port and click “OK” button